Lake Elsinore and Canyon Lake

Watersheds Nutrient TMDL Monitoring

2018-2019 Annual Report - DRAFT

Prepared for:

Lake Elsinore & San Jacinto Watersheds Project Authority

11615 Sterling Avenue

Riverside, California 92503

Prepared by:

Wood Environment and Infrastructure Inc.

(formerly Amec Foster Wheeler)

9210 Sky Park Court

Suite 200

San Diego, CA 92123

August 13, 2019

TABLE OF CONTENTS

Page

1.0	Introduction	7

1.1	Background	7

2.0	San Jacinto River Watershed-Wide Monitoring	9

2.1	Summary of 2018-2019 Wet Weather Watershed Monitoring and Nutrient Loads	9

2.2	Monitoring Strategy	10

2.3	Monitoring Stations and Stream Gauge Locations	10

2.4	Stream Gauge Records	12

2.5	Sampling Strategy	16

2.6	San Jacinto Watershed Monitoring Events	16

2.7	San Jacinto Watershed Annual Water Quality Summary	17

2.7.1	Summary of Monitoring Data – Salt Creek at Murrieta Road	18

2.7.2	Summary of Monitoring Data – San Jacinto River at Goetz Road	22

2.7.3	Summary of Monitoring Data – San Jacinto River at Ramona Expressway	28

2.7.4	Summary of Monitoring Data – Cranston Guard Station	28

2.7.5	Summary of Monitoring Data – Canyon Lake Spillway	29

2.8	San Jacinto Watershed Rainfall Records	35

3.0	In-Lake Monitoring	37

3.1	Background	37

3.2	Lake Elsinore Monitoring	38

3.2.1	Sampling Station Locations and Frequency	38

3.2.2	Sampling Methods	40

3.2.3	Water Quality Summary	41

3.3	Canyon Lake Monitoring	55

3.3.1	Sampling Station Locations and Frequency	55

3.3.2	Sampling Methods	57

3.3.3	Water Quality Summary	58

3.4	Satellite Imagery	76

4.0	References	90

LIST OF APPENDICES

Appendix A Watershed Monitoring Data

Appendix B QUARTERLY LAKE MONITORING REPORTS

Appendix C Lake Monitoring Analytical Reports

Appendix D Satellite Data Reports

Appendix E Current DAta in Historical context

LIST OF TABLES

Table 2-1. Summary of 2018-2019 Monitoring	10

Table 2-2. San Jacinto River Watershed Monitoring Stations	11

Table 2-3. Summary of Stream Gauge Data (July 2018 through June 2019)	13

Table 2-4. Water Quality Concentrations at Salt Creek at Murrieta Road	20

Table 2-5. Water Quality Event and Annual Loads at Salt Creek at Murrieta Road	20

Table 2-6. Water Quality Concentrations at San Jacinto River at Goetz Road	25

Table 2-7. Water Quality Event and Annual Loads at San Jacinto River at Goetz Road	25

Table 2-8. Water Quality EMCs at Canyon Lake Spillway	32

Table 2-9. Water Quality Event and Annual Loads at Canyon Lake Spillway	32

Table 2-10. San Jacinto River Watershed Rainfall Gauges	35

Table 2-11. Summary Rainfall Data (July 2018 to June 2019)	36

Table 3-1.  Lake Elsinore Monitoring Stations	38

Table 3-2.  2018-2019 In-lake Analytical Constituents and Methods for Lake Elsinore	40

Table 3-3. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2018-2019 Monthly Means for Each Site (July – Dec 2018)	41

Table 3-4. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2018-2019 Monthly Means for Each Site (Feb – June 2019)	42

Table 3-5. In-Situ Water Quality Parameter Measurements in Lake Elsinore - Annual Mean Statistics for Each Site	43

Table 3-6.  Monthly Analytical Chemistry Results for Lake Elsinore in 2018-2019	50

Table 3-7. Analytical Chemistry Summary for Lake Elsinore – Annual Mean Statistics (2018-2019)	51

Table 3-8.  Canyon Lake Sampling Station Locations	55

Table 3-9.   In-lake Analytical Constituents and Methods for Canyon Lake (2018-2019)	58

Table 3-10. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (Aug – Dec 2018)	60

Table 3-11. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (Feb – June 2019)	60

Table 3-12. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (Aug – Dec 2018)	61

Table 3-13. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (Feb – June 2019)	61

Table 3-14. In-Situ Water Quality Parameter Measurements for Canyon Lake - Annual Mean Statistics for Each Site (Aug 2018 – June 2019)	62

Table 3-15. Analytical Chemistry Results for Canyon Lake - Monthly Depth-Integrated Results (Aug – Dec 2018)	69

Table 3-16. Analytical Chemistry Results for Canyon Lake- Monthly Depth-Integrated Results (Feb – June 2019)	70

Table 3-17. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Each Site in the Main Basin	71

Table 3-18. Analytical Chemistry Results for Canyon Lake- Annual Mean Statistics for Each Site in the East Basin	72

Table 3-19. Analytical Chemistry Results for Canyon Lake- Annual Mean Statistics for Both Main and East Basins	73

LIST OF FIGURES

Figure 2-1. San Jacinto River Watershed Monitoring Stations	11

Figure 2-2. Site 3 – Salt Creek at Murrieta Road – Daily Stream Gauge Records	13

Figure 2-3. Site 4 – San Jacinto River at Goetz Road – Daily Stream Gauge Records	14

Figure 2-4. Site 30 – Canyon Lake Spillway – Daily Stream Gauge Records	14

Figure 2-5. Site 6 – San Jacinto River at Ramona Expressway – Daily Stream Gauge Records	15

Figure 2-6. Site 1 – San Jacinto River at Cranston Guard Station – Daily Stream Gauge Records	15

Figure 2-7. Storm Event at Salt Creek at Murrieta Road (November 29-30, 2018)	18

Figure 2-8. Storm Event at Salt Creek at Murrieta Road (December 5-7, 2018)	19

Figure 2-9. Storm Event at Salt Creek at Murrieta Road (January 31, 2019 – February 4, 2019)	19

Figure 2-10. Hydrograph of First Storm Event at Salt Creek at Murrieta Road (November 29-30, 2018)	21

Figure 2-11. Hydrograph of Second Storm Event at Salt Creek at Murrieta Road (December 5-7, 2018)	22

Figure 2-12. Hydrograph of Third Storm Event at Salt Creek at Murrieta Road (January 31, 2019 – February 4, 2019)	22

Figure 2-13. Storm Event at San Jacinto River at Goetz Road (November 29, 2018 – December 2, 2018)	23

Figure 2-14. Storm Event at San Jacinto River at Goetz Road (December 6-7, 2018)	24

Figure 2-15. Storm Event at San Jacinto River at Goetz Road (January 31, 2019 – February 4, 2019)	24

Figure 2-16. Hydrograph of First Storm Event at San Jacinto River at Goetz Road  (November 29, 2018 – December 2, 2018)	26

Figure 2-17. Hydrograph of Second Storm Event at San Jacinto River at Goetz Road  (December 6-7, 2018)	27

Figure 2-18. Hydrograph of Third Storm Event at San Jacinto River at Goetz Road (January 31, 2019 – February 4, 2019)	28

Figure 2-19. Storm Event at Canyon Lake Spillway (January 16-20, 2019)	30

Figure 2-20. Storm Event at Canyon Lake Spillway (January 31, 2019 – February 6, 2019)	30

Figure 2-21. Storm Event at Canyon Lake Spillway (February 14, 2019 – February 18, 2019)	31

33

Figure 2-22. Hydrograph of First Storm Event at Canyon Lake Spillway (January 16-20, 2019)	33

33

Figure 2-23. Hydrograph of Second Storm Event at Canyon Lake Spillway (January 31, 2019 – February 6, 2019)	33

34

Figure 2-24. Hydrograph of Third Storm Event at Canyon Lake Spillway (February 14, 2019 – February 18, 2019)	34

Figure 2-25. Canyon Lake Level at Railroad Canyon Dam Spillway	34

39

Figure 3-1. Lake Elsinore Sampling Locations	39

Figure 3-2. Water Column Mean Dissolved Oxygen (DO) Rolling Average – Lake Elsinore – Site LE02	44

Figure 3-3. In-Situ Physical Water Quality Parameters - Lake Elsinore - Site LE01	45

Figure 3-4a. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE02	46

Figure 3-4b. In-Situ Secchi Depth Measurements - Lake Elsinore Site LE02	47

Figure 3-5. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE03	48

Figure 3-6. Lake Elsinore Analytical Chemistry – Depth-Integrated Means  (July 2018-June 2019)	52

Figure 3-7. Lake Elsinore Analytical Chemistry – Rolling Averages  (July 2018 – June 2019)	53

Figure 3-8. Lake Elsinore Analytical Chemistry – Depth-Integrated and Surface Chlorophyll-a	54

Figure 3-9. Canyon Lake Sampling Locations	56

Figure 3-10a. Mean In-Situ Physical Water Quality Parameters – Canyon Lake Main Basin	63

Figure 3-10b. Mean In-Situ Physical Water Quality Parameters - Canyon Lake East Basin	64

Figure 3-11. Rolling Average Concentrations of Dissolved Oxygen in the Epilimnion and Hypolimnion of Canyon Lake	65

Values are calculated by averaging the value from each event with the previous five event values (i.e. one year of data) to obtain a rolling average.	65

Figure 3-12. Rolling Average Concentration of Dissolved Oxygen Across the Full Vertical Water Column in Canyon Lake	65

Figure 3-13. In- Situ Secchi Depth Measurements – Main and East Basins	66

Figure 3-14. Canyon Lake during February 2019 Monitoring Event Following a Series of Large Storm Events in January and February.  Note highly turbid water.	66

Figure 3-15. Canyon Lake Analytical Chemistry – Depth-Integrated Means	74

Figure 3-16. Canyon Lake Analytical Chemistry – Depth-Integrated Chlorophyll-a	75

Figure 3-17. Canyon Lake Analytical Chemistry- Rolling Averages	76

Figure 3-18.  Satellite Imagery of Chlorophyll-a Concentrations in Lake Elsinore	79

Figure 3-19.  Satellite Imagery of Turbidity Concentrations in Lake Elsinore	81

Figure 3-20.  Satellite Imagery of Chlorophyll-a Concentrations in Canyon Lake	83

Figure 3-21.  Satellite Imagery of Turbidity Measurements Canyon Lake	85

Figure 3-22.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Lake Elsinore Relative to Field Collected Samples	87

Figure 3-23.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Canyon Lake Relative to Field Collected Samples	88

Acronyms and Abbreviations

| µg/L | micrograms per liter |
| --- | --- |
| µS/cm | microSiemens per centimeter |
| Basin Plan | Water Quality Control Plan for the Santa Ana River Basin |
| CCC | criterion continuous concentration |
| cf | cubic feet |
| cfs | cubic feet per second |
| CMC | criterion maximum concentration |
| DO | dissolved oxygen |
| EVMWD | Elsinore Valley Municipal Water District |
| EMC | event mean concentration |
| Forest Service | San Bernardino Nation Forest Service |
| FY | fiscal year |
| kg | kilogram |
| LESJWA | Lake Elsinore and San Jacinto Watersheds Authority |
| LA | load allocation |
| Mgal | million gallons of water |
| mg/L | milligrams per liter |
| NPDES | National Pollutant Discharge Elimination System |
| NWS | National Weather Service |
| ND | non-detect |
| QAPP | Quality Assurance Project Plan |
| RCFC&WCD | Riverside County Flood Control and Water Conservation District |
| RWQCB | Regional Water Quality Control Board, Santa Ana Region |
| SAWPA | Santa Ana Watershed Project Authority |
| TDS | Total Dissolved Solids |
| TMDL Task Force | Lake Elsinore and Canyon Lake TMDL Task Force |
| TMDL | Total Maximum Daily Load |
| US EPA | U.S. Environmental Protection Agency |
| USGS | United States Geological Survey |
| WLA | waste load allocation |

This page intentionally left blank.

# Introduction

The following document summarizes results of compliance monitoring required in support of the Lake Elsinore and Canyon Lake Nutrient Total Maximum Daily Load (TMDL) for the 2018-2019 fiscal year (FY). The monitoring was performed according to the Lake Elsinore & Canyon Lake Nutrient TMDL Monitoring Quality Assurance Project Plan (QAPP) (Amec Foster Wheeler, September 2016), and the associated Compliance Monitoring Work Plan (Haley & Aldrich, Inc., July 2016).

## Background

Lake Elsinore is a natural freshwater lake in southern California that provides a variety of natural habitats for terrestrial and aquatic species.  The beneficial uses of the lake include water contact recreation (REC1), non-water contact recreation (REC2), commercial and sportfishing (COMM), warm freshwater habitat (WARM), wildlife habitat (WILD), and rare, threatened or endangered species (RARE).  Canyon Lake was constructed in 1928 as the Railroad Canyon Reservoir.  It is located approximately two miles upstream of Lake Elsinore and water spilled from Canyon Lake is a main source of water for Lake Elsinore during wet years.  The beneficial uses of Canyon Lake include municipal and domestic water supply (MUN), agricultural supply (AGR), groundwater recharge (GWR), body contact recreation (REC1), non-body contact recreation (REC2), commercial and sportfishing (COMM), warm freshwater aquatic habitat (WARM), and wildlife habitat (WILD).  The beneficial uses of COMM and RARE in Lake Elsinore and COMM in Canyon Lake were recently approved by the California Regional Water Quality Control Board, Santa Ana Region (RWQCB) as a Basin Plan Amendment under tentative resolution R8-2017-0019 on June 16, 2017.

In 1994, Lake Elsinore and Canyon Lake were first listed by the RWQCB on its Clean Water Act Section 303(d) list of impaired waterbodies.  Both lakes remain on the latest approved 303(d) list finalized in 2010.  Impairments identified for these waters included excessive levels of nutrients in both lakes, as well as organic enrichment/low dissolved oxygen (DO), sedimentation/siltation, unknown causes of toxicity in Lake Elsinore, and high bacterial indicators in Canyon Lake.  The Clean Water Act Section 303(d) requires the development and implementation of a TMDL for waters that do not or are not expected to meet water quality standards (beneficial uses, water quality objectives). In 2000, the RWQCB initiated the development of TMDLs for nutrients for Lake Elsinore and Canyon Lake.

In December 2004, the RWQCB adopted amendments to the Water Quality Control Plan for the Santa Ana River Basin (Basin Plan) to incorporate TMDLs for nutrients in Canyon Lake and Lake Elsinore.  The Regional Board adopted the Resolution, and it was subsequently approved by the U.S. Environmental Protection Agency (US EPA) on September 30, 2005.  The Basin Plan Amendment specifies, among other things, monitoring recommendations to track compliance with the TMDL and associated waste load allocations (WLAs) and monitoring to measure compliance towards in-lake numeric water quality targets. Numeric targets have been established and incorporated in the TMDL for nutrients (total nitrogen, phosphorous, and ammonia), DO, and chlorophyll-a; however, the ultimate compliance goal for beneficial uses in both lakes is to reduce enhanced eutrophication, which can negatively affect biological communities, result in fish kills, and impact recreational use. The recommendations outlined in RWQCB Resolution No. R8-2004-0037 required stakeholders to develop management plans and conduct long-term monitoring and implementation programs aimed at reducing nutrient loads to Lake Elsinore and Canyon Lake.  Task 4 of the adopted Lake Elsinore and Canyon Lake TMDL Amendment required stakeholders to prepare and implement a Nutrient Monitoring Program. The program was to include the following:

- A watershed-wide monitoring program to determine compliance with interim and/or final nitrogen and phosphorus allocations; compliance with the nitrogen and phosphorus TMDL, and load allocations (LAs), including WLAs.

- A Lake Elsinore in-lake nutrient monitoring program to determine compliance with interim and final nitrogen, phosphorus, chlorophyll-a, and DO numeric targets.

- A Canyon Lake in-lake nutrient monitoring program to determine compliance with interim and final nitrogen, phosphorus, chlorophyll-a, and DO numeric targets.

- An draft annual report summarizing the data collected for the year and evaluating compliance with the TMDL, due August 15 of each year.

Since August 2001, the Lake Elsinore and San Jacinto Watershed Authority (LESJWA) has been working with local stakeholders and the Santa Ana Regional Water Quality Control Board to identify the source of nutrients impairing each lake and evaluate the impacts to water quality and beneficial uses incurred from nutrient sources.

At that time, LESJWA contracted with the State to serve as a neutral facilitator for the RWQCB to assist in formation of a TMDL workgroup and assisting the workgroup in participating with the RWQCB in the development and definition of the TMDLs.

With formal adoption of the Lake Elsinore and Canyon Lake nutrient TMDLs on December 20, 2004, stakeholders named in the TMDLs began the process to create a formal cost sharing body, or Task Force to implement a number of tasks defined within the TMDLs.

In November 2006, stakeholders finalized an agreement to form the Lake Elsinore and Canyon Lake TMDL Task Force.  The TMDL Task Force consists of representatives from local cities, Riverside County, agriculture and dairy, and the regulatory community.  At the request of the stakeholders and RWQCB, LESJWA staffed by the Santa Ana Watershed Project Authority (SAWPA) serves as administrator of the Task Force and oversees the TMDL development process for Lake Elsinore and Canyon Lake.

LESJWA, in support of the TMDL Task Force, provided funding to meet the requirement of the TMDL by developing a single comprehensive watershed-wide nutrient Monitoring Plan. The Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan was approved by the RWQCB in March 2006 and subsequently implemented by the TMDL Task Force in April 2006 through October 2012.  During this time frame, the in-lake monitoring for both lakes was conducted through the Elsinore Valley Municipal Water District (EVMWD) National Pollutant Discharge Elimination System (NPDES) compliance program (Order No. R8-2005-0003 for NPDES No. CA8000027 for the Regional Water Reclamation Plant, Lake Elsinore, Riverside County approved March 4, 2005).  On October 26, 2012 the Regional Board issued a resolution (Resolution No. R8-2012-0052) granting the TMDL Task Force a temporary suspension of in-lake TMDL monitoring programs to achieve cost savings that were applied to implementing lake improvement projects aimed at reducing nutrient impacts in Canyon Lake and Lake Elsinore. Therefore, the Lake Elsinore and Canyon Lake Nutrient TMDL field compliance monitoring was not conducted during the 2013-2014 and 2014-2015 FY cycles.

The in-lake water quality monitoring for both lakes was resumed in July 2015 as Phase II of the Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Program moving forward.  The results of the 2018-2019 FY in-lake and watershed monitoring efforts are summarized herein.

# San Jacinto River Watershed-Wide Monitoring

The study design for Phase II of the San Jacinto River Watershed Monitoring Program is to continue to determine nutrient loading into Canyon Lake and Lake Elsinore from upstream watershed sources to add to the historical monitoring data set to evaluate long-term trends. The primary objectives of the Phase II San Jacinto River Watershed Monitoring Program are as follows:

- Determine the total nutrient loads into Lake Elsinore and Canyon Lake from their tributaries (i.e., the San Jacinto River, Salt Creek, and Cottonwood Creek).

- Determine the total nutrient load from various sources categorized by land use types, namely, agricultural, urban runoff, and open space sources which drain into the above-mentioned tributaries.

- Provide water quality data for watershed model updates.

- Provide water quality data to evaluate TMDL compliance with WLAs and LAs.

Watershed monitoring and reporting was performed by Alta Environmental of San Diego, California.

## Summary of 2018-2019 Wet Weather Watershed Monitoring and Nutrient Loads

A summary of the water quality monitoring data for each of the five monitoring locations for the period of July 1, 2018 through June 30, 2019, is presented in Table 2-1 below. A more detailed account, including storm hydrographs and event loads are presented in Section 2.7 for each monitoring location. The complete set of water quality data, including water quality field measurements is included in Appendix A.

Table 2-1. Summary of 2018-2019 Monitoring

| Number and Location Description | Total Annual Flow a (Mgal) | Annual Event Mean Storm Concentration (mg/L) |  | Estimated Annual Load (kg) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Total Nitrogen | Total Phosphorus | Total Nitrogen | Total Phosphorus |
| Site 3 - Salt Creek at Murrieta Road (USGS 11070465) | 1,394 | 2.37 | 0.42 | 12,213 | 2,266 |
| Site 4 - San Jacinto River at Goetz Road (USGS 11070365) | 3,208 | 1.70 | 0.61 | 20,457 | 7,409 |
| Site 6 - San Jacinto River at Ramona Expressway b (USGS 11070210) | 12 | Not Measured c | Not Measured c | Not Measured c | Not Measured c |
| Site 30 - Canyon Lake Spillway c (USGS 11070500) | 5,893 | 1.40 | 0.19 | 32,832 | 5,416 |

a - Flow data after 10/29/2018 are provisional and may be subject to change.

b - No flows originating from the upper watershed were observed at the TMDL monitoring location just downstream of Mystic Lake, only local flows were observed, and no sampling was conducted.

c –The USGS stream gauge at Site 30 (USGS 11070500) is located downstream of Canyon Lake on the San Jacinto River close to the river entrance to Lake Elsinore.  This downstream location is influenced by local urban runoff and groundwater seepage in addition to the flows from Canyon Lake.

Mgal = million gallons; 1 million gallons = 133,680 cubic feet; mg/L = milligrams per liter; kg = kilograms; USGS = United States Geological Survey.

## Monitoring Strategy

Phase II of the San Jacinto River Watershed Monitoring Program follows the guidelines detailed in the Lake Elsinore and Canyon Lake Nutrient TMDL Compliance Monitoring Plan.  The Phase II San Jacinto River Watershed Monitoring Program sampling activities during the 2018-2019 monitoring period included collection of samples during three storm events at the designated monitoring stations throughout the San Jacinto River Watershed.

## Monitoring Stations and Stream Gauge Locations

To monitor TMDL compliance, five sampling stations were carefully selected to reflect various types of land uses within the San Jacinto River Watershed. These locations have been monitored since 2006.  Sampling station locations were deliberately set up to be within the vicinity of United States Geological Survey (USGS) or the Riverside County Flood Control and Water Conservation District (RCFC&WCD) stream gauge stations.  The sampling stations are listed in Table 2-2 below and shown on Figure 2-1.

Three of the five sites (Station IDs 745, 759, and 741) were selected because they are indicative of inputs to Canyon Lake originating from the main stem of the San Jacinto River, Salt Creek, and the watershed above Mystic Lake. The sampling location along the San Jacinto River at Ramona Expressway (Station 741) is located downgradient of Mystic Lake, an area of land subsidence. Flow has not been observed at this location since a strong El Niño event in the mid-1990s. Because of the active subsidence, this monitoring station is not expected to flow except under extremely high rainfall conditions.

Table 2-2. San Jacinto River Watershed Monitoring Stations

| Station ID | USGS Station ID | Agency | Site Number and Location Description |
| --- | --- | --- | --- |
| 745 | 11070465 | USGS | Site 3 - Salt Creek at Murrieta Road |
| 759 | 11070365 | USGS | Site 4 - San Jacinto River at Goetz Road |
| 741 | 11070210 | USGS | Site 6 - San Jacinto River at Ramona Expressway |
| 841 | 11070500 | USGS | Site 30 - Canyon Lake Spillway |
| 792 | 11069500 | RCFC&WCD or USGS | Site 1 - San Jacinto River at Cranston Guard Station |

Figure 2-1. San Jacinto River Watershed Monitoring Stations

The fourth site, located below the Canyon Lake Dam (Station ID 841) is indicative of loads entering Lake Elsinore from Canyon Lake and the upstream watershed when the water level overtops the Railroad Canyon Dam Spillway. The Railroad Canyon Dam Spillway elevation at Canyon Lake is 1,381.76 feet. Samples are collected from this location during storm events that create lake levels that overtop the dam spillway elevation.  The Canyon Lake level is publicly available at the following website:

http://www.evmwd.com/about/departments/public/lake.asp

The fifth site at the Cranston Guard Station site on the San Jacinto River (Station 792) was only monitored between 2007 and 2011 by the San Bernardino National Forest Service (Forest Service), in accordance with their agreement for in-lieu obligations to the Task Force. This work, however, was dependent on sufficient funds being allocated by Congress to complete the work.  In 2012, the Forest Service pulled out of the Task Force and no longer provides monitoring support.

## Stream Gauge Records

The USGS and RCFC&WCD monitor stream flow from several gauging stations in the San Jacinto River Watershed. Stream gauging stations maintained and operated for Phase II of the San Jacinto Watershed Monitoring Program are shown in Figure 2-1 and identified in Table 2-2.

The data record captured per USGS stream gauge is publicly available at the following website:

http://waterdata.usgs.gov/ca/nwis/current/?type=flow

A summary of the stream gauge data recorded at each of the stations with measured flow for the monitoring period of July 1, 2018 through June 30, 2019 is presented in Table 2-3 and visually presented in Figure 2-2 through Figure 2-6.  The mean monthly flows reported in Table 2-3 characterize the average instantaneous flow rate at the USGS station during both dry and wet weather conditions.  The flow data are downloaded from the USGS website and are considered provisional for approximately six months; therefore, flow data presented after October 29, 2018, in this report are provisional.  The provisional data provided by the USGS are subject to change and are not citable until reviewed and approved by the USGS.  The complete set of stream gauge data is included as Appendix A.

Table 2-3. Summary of Stream Gauge Data (July 2018 through June 2019)

| July 2017-June 2018 Mean Monthly Flow (cfs) a | Site 3 - Salt Creek at Murrieta Road (11070465) | Site 4 - San Jacinto River at Goetz Road (11070365) | Site 6 - San Jacinto River at Ramona Expressway b (11070210) | Site 30 - Canyon Lake Spillway (11070500) | Site 1 - San Jacinto River at Cranston Guard Station (11069500) |
| --- | --- | --- | --- | --- | --- |
| July | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 |
| August | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| September | 0.00 | 25.65 | 0.00 | 0.00 | 0.00 |
| October | 1.65 | 2.53 | 0.00 | 0.06 | 0.00 |
| November | 0.46 | 4.37 | 0.00 | 0.37 | 0.00 |
| December | 4.05 | 11.07 | 0.00 | 1.77 | 0.58 |
| January | 8.24 | 16.96 | 0.00 | 42.01 | 14.13 |
| February | 56.38 | 91.53 | 0.58 | 246.71 | 168.92 |
| March | 4.07 | 12.77 | 0.08 | 25.39 | 81.77 |
| April | 0.00 | 0.63 | 0.00 | 1.03 | 27.98 |
| May | 0.25 | 4.47 | 0.00 | 0.62 | 23.38 |
| June | 0.00 | 0.03 | 0.00 | 0.21 | 9.26 |
| Mean Annual Flow (cfs) | 5.91 | 13.60 | 0.05 | 25.09 | 26.17 |

Notes:

a - This value characterizes the average instantaneous flow rate at the USGS station during both dry and wet weather conditions in a given month. Flow data after 10/29/2018 are provisional and may be subject to change.

b - No flows originating from the upper watershed were observed at the TMDL monitoring location just downstream of Mystic Lake, only local flows were observed.

cfs = cubic feet per second.

Figure 2-2. Site 3 – Salt Creek at Murrieta Road – Daily Stream Gauge Records

Figure 2-3. Site 4 – San Jacinto River at Goetz Road – Daily Stream Gauge Records

Figure 2-4. Site 30 – Canyon Lake Spillway – Daily Stream Gauge Records

Figure 2-5. Site 6 – San Jacinto River at Ramona Expressway – Daily Stream Gauge Records

Figure 2-6. Site 1 – San Jacinto River at Cranston Guard Station – Daily Stream Gauge Records

## Sampling Strategy

Phase II of the San Jacinto River Watershed Monitoring Program includes collecting water quality samples during three storm events at the designated monitoring stations throughout the San Jacinto River Watershed. Throughout the wet weather monitoring period from October 1, 2018 to May 31, 2019, the National Weather Service (NWS) forecasts were monitored to determine when storm events met the mobilization criteria. The mobilization criteria for sampling requires a NWS quantitative precipitation forecast greater than a 1.0-inch forecast within 24 hours from October 1 through December 31, and greater than an 0.5-inch forecast within 24 hours from January 1 through May 31.

Flow-weighted composite samples were collected during three storm events at the designated monitoring stations. Discrete sample aliquots were collected over the rising limb (increasing flow) and the falling limb (decreasing flow) of the hydrograph using automatic sampling equipment (e.g., ISCO autosamplers). The first sample aliquot was taken at or shortly after the time that storm water runoff began, and each subsequent aliquot of equal volume was collected at intervals of approximately 1 to 2 hours across the hydrograph, depending on the forecasted size of the storm event.  Flow rates and volumes were based on data from USGS stream gauges located near the sampling stations. Upon completion of sampling, field teams downloaded the USGS flow data and subsampled each discrete sample to create a single flow-weighted composite sample for laboratory analysis.

The following protocols were applied:

- Sampling commenced once flow was established in the channel.

- Field measurements (temperature, pH, conductivity, dissolved oxygen, and turbidity) were recorded in the field using portable calibrated YSI multi-parameter meters, or equivalent.

- Biochemical Oxygen Demand and Chemical Oxygen Demand were analyzed for the first discrete grab sample only.

Sampling and analysis followed the guidelines detailed in the Lake Elsinore and Canyon Lake Nutrient TMDL Compliance Monitoring Plan. More detail regarding the sampling approach (e.g., compositing, sample naming conventions) are described in the Lake Elsinore and Canyon Lake Nutrient TMDL Compliance QAPP.

Samples for all analytical chemistry measurements were submitted to Babcock Laboratories Inc. located in Riverside, California.

## San Jacinto Watershed Monitoring Events

Water quality samples were collected during five storm events that met the mobilization criteria during the wet weather monitoring period from October 1, 2018 to May 31, 2019, which included at total of three events for each station.

The first monitoring event occurred on November 29, 2018 through December 2, 2018. Water quality samples were collected at Salt Creek at Murrieta Road (Station ID 745) and San Jacinto River at Goetz Road (Station ID 759).  A peak flow of 42 cubic feet per second (cfs) was recorded at Salt Creek at Murrieta Road (Station ID 745) and a peak flow of 225 cfs was recorded at San Jacinto River at Goetz Road (Station ID 759).  No flows exited Canyon Lake during the monitoring event (i.e., the water level in Canyon Lake did not crest the spillway) and no flows were recorded at the San Jacinto River at Ramona Expressway (Station ID 741).  A total of 0.59 to 1.23 inches of rainfall was recorded in the region during this storm (RCFCWCD 2019).

The second monitoring event occurred on December 5, 2018 through December 7, 2018. Water quality samples were collected at Salt Creek at Murrieta Road (Station ID 745) and San Jacinto River at Goetz Road (Station ID 759).  A peak flow of 244 cubic feet per second (cfs) was recorded at Salt Creek at Murrieta Road (Station ID 745) and a peak flow of 668 cfs was recorded at San Jacinto River at Goetz Road (Station ID 759).  No flows exited Canyon Lake during the monitoring event, and no flows were recorded at the San Jacinto River at Ramona Expressway (Station ID 741).  A total of 1.15 to 1.46 inches of rainfall was recorded in the region during this storm (RCFCWCD 2019).

The third monitoring event occurred on January 16, 2019 through January 20, 2019. Water quality samples were collected at Canyon Lake Spillway (Station ID 841). A peak flow of 537 cfs was recorded at Canyon Lake Spillway (Station ID 841). No flows were recorded at the San Jacinto River at Ramona Expressway (Station ID 741).  A total of 1.21 to 2.62 inches of rainfall was recorded in the region during this storm (RCFCWCD 2019).

The fourth monitoring event occurred on January 31, 2019 through February 6, 2019. Water quality samples were collected at Salt Creek at Murrieta Road (Station ID 745), San Jacinto River at Goetz Road (Station ID 759), and Canyon Lake Spillway (Station ID 841).   A peak flow of 268 cubic feet per second (cfs) was recorded at Salt Creek at Murrieta Road (Station ID 745), a peak flow of 774 cfs was recorded at San Jacinto River at Goetz Road (Station ID 759), and a peak flow of 788 cfs was recorded at Canyon Lake Spillway (Station ID 841). No flows were recorded at the San Jacinto River at Ramona Expressway (Station ID 741).  A total of 1.83 to 2.75 inches of rainfall was recorded in the region during this storm (RCFCWCD 2019).

The fifth monitoring event occurred on February 14, 2019 through February 18, 2019. Water quality samples were collected at Canyon Lake Spillway (Station ID 841). A peak flow of 6,380 cfs was recorded at Canyon Lake Spillway (Station ID 841). Flows from the local area were observed at the San Jacinto River at Ramona Expressway (Station ID 741) beginning on February 14, 2019, however, no flows originating from the upper watershed were observed and no flows exited Mystic Lake. A total of 2.79 to 3.82 inches of rainfall was recorded in the region during this storm (RCFCWCD 2019).

## San Jacinto Watershed Annual Water Quality Summary

A summary of watershed water quality monitoring data for each of the five monitoring locations for the monitoring period of July 1, 2018 through June 30, 2019, is presented below. The complete set of water quality data for the monitoring period is included as Appendix A. Included with each summary of the monitoring data are the concentrations for each analyte. Also included are the estimated storm event loads and annual loads for each analyte.

### Summary of Monitoring Data – Salt Creek at Murrieta Road

Water quality samples were collected during three storm events at Salt Creek at Murrieta Road (Station ID 745) during the wet weather monitoring period from October 1, 2018 to May 31, 2019.

During the storm event on November 29, 2018 through November 30, 2018, a total of 22 discrete samples were collected across the hydrograph at one-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070465), flow for the storm event was estimated at 1,200,123 cubic feet (cf) or 9.0 million gallons (Mgal).

During the storm event on December 5, 2018 through December 7, 2018, a total of 33 discrete samples were collected across the hydrograph at one-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070465), flow for the storm event was estimated at 10,735,389 cf or 80.3 Mgal.

During the storm event on January 31, 2019 through February 4, 2019, a total of 35 discrete samples were collected across the hydrograph at two-hour intervals for the first 11 discrete samples and at three-hour intervals for the final 24 discrete samples. A single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070465), flow for the storm event was estimated at 28,634,769 cf or 214.2 Mgal.

Photos taken during the storm events are provided in Figure 2-7, Figure 2-8, and Figure 2-9.

Figure 2-7. Storm Event at Salt Creek at Murrieta Road (November 29-30, 2018)

Figure 2-8. Storm Event at Salt Creek at Murrieta Road (December 5-7, 2018)

Figure 2-9. Storm Event at Salt Creek at Murrieta Road (January 31, 2019 – February 4, 2019)

Event and annual mean concentrations for each analyte are presented in Table 2-4.  Event and annual loads for each analyte are presented in Table 2-5.  Concentrations for nutrients for the three storm events range from 1.9 to 2.7 milligrams per liter (mg/L) for total nitrogen, and 0.31 to 0.49 mg/L for total phosphorus (Table 2-4).  Based on flow data provided by the nearby USGS stream gauge (Station ID 11070465), the total annual flow was estimated at 186,298,254 cf or 1,393.6 Mgal for the period of July 1, 2018 through June 30, 2019. No dry weather flows enter Canyon Lake from Salt Creek at Murrieta Road (Station ID 745) and the storm flows account for the estimated annual load of nutrients. The estimated annual nutrient load was calculated to be 12,213 kg for total nitrogen and 2,226 kg for total phosphorus (Table 2-5) for the period of July 1, 2018 through June 30, 2019.

Table 2-4. Water Quality Concentrations at Salt Creek at Murrieta Road

| Analyte | Units | Event 1 | Event 2 | Event 3 | Annual Mean | Annual Geomean |
| --- | --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | mg/L | 0.32 | 0.25 | ND(<0.48) | 0.270 | 0.268 |
| Chemical Oxygen Demand | mg/L | 64 | 55 | 35 | 51 | 50 |
| Kjeldahl Nitrogen | mg/L | 1.5 | 1.9 | 1.5 | 1.6 | 1.6 |
| Nitrate as N | mg/L | 0.99 | 0.79 | 0.35 | 0.71 | 0.65 |
| Nitrite as N | mg/L | ND(<0.059) | ND(<0.059) | ND(<0.059) | 0.030 | 0.030 |
| Organic Nitrogen | mg/L | 1.2 | 1.7 | 1.5 | 1.5 | 1.5 |
| Total Nitrogen | mg/L | 2.5 | 2.7 | 1.9 | 2.4 | 2.3 |
| Total Phosphorus | mg/L | 0.31 | 0.49 | 0.46 | 0.42 | 0.41 |
| Ortho Phosphate Phosphorus | mg/L | 0.38 | 0.14 | 0.21 | 0.24 | 0.22 |
| Total Dissolved Solids | mg/L | 92 | 530 | 350 | 324 | 257 |
| Total Hardness | mg/L | 37 | 240 | 150 | 142 | 110 |
| Total Suspended Solids | mg/L | 28 | 130 | 78 | 79 | 66 |

Table 2-5. Water Quality Event and Annual Loads at Salt Creek at Murrieta Road

| Analyte | Units | Load Event 1 | Load Event 2 | Load Event 3 | Annual Load |
| --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | kg | 11 | 76 | 195 | 1,396 |
| Chemical Oxygen Demand | kg | 2,175 | 16,720 | 28,380 | 259,104 |
| Kjeldahl Nitrogen | kg | 51 | 578 | 1,216 | 8,585 |
| Nitrate as N | kg | 34 | 240 | 284 | 3,487 |
| Nitrite as N | kg | 1 | 9 | 24 | 156 |
| Organic Nitrogen | kg | 41 | 517 | 1,216 | 7,826 |
| Total Nitrogen | kg | 85 | 821 | 1,541 | 12,213 |
| Total Phosphorus | kg | 11 | 149 | 373 | 2,266 |
| Ortho Phosphate Phosphorus | kg | 13 | 43 | 170 | 1,230 |
| Total Dissolved Solids | kg | 3,127 | 161,116 | 283,796 | 1,785,043 |
| Total Hardness | kg | 1,257 | 72,958 | 121,627 | 783,189 |
| Total Suspended Solids | kg | 952 | 39,519 | 63,246 | 428,339 |

Hydrographs with flow-weighted sample aliquot times are provided in Figure 2-7, Figure 2-8, and Figure 2-9. The figures were developed based on flow data provided by the nearby USGS stream gauge (Station ID 11070465).

Figure 2-10. Hydrograph of First Storm Event at Salt Creek at Murrieta Road
(November 29-30, 2018)

Figure 2-11. Hydrograph of Second Storm Event at Salt Creek at Murrieta Road
(December 5-7, 2018)

Figure 2-12. Hydrograph of Third Storm Event at Salt Creek at Murrieta Road
(January 31, 2019 – February 4, 2019)

### Summary of Monitoring Data – San Jacinto River at Goetz Road

Water quality samples were collected during two storm events at San Jacinto River at Goetz Road (Station ID 759) during the wet weather monitoring period from October 1, 2018 to May 31, 2019.

During the storm event on November 29, 2018 through December 2, 2018, a total of 33 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070365), flow for the storm event was estimated at 12,865,743 cf or 96.2 Mgal.

During the storm event on December 6, 2018 through December 7, 2018, a total of 27 discrete samples were collected across the hydrograph at one-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070365), flow for the storm event was estimated at 28,109,421 cf or 210.3 Mgal.

During the storm event on January 31, 2019 through February 4, 2019, a total of 33 discrete samples were collected across the hydrograph at two-hour intervals for the first 11 discrete samples and at three-hour intervals for the final 22 discrete samples. A single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070365), flow for the storm event was estimated at 53,268,723 cf or 398.5 Mgal.

Photos taken during the storm events are provided in Figure 2-13, Figure 2-14, and Figure 2-15.

Figure 2-13. Storm Event at San Jacinto River at Goetz Road (November 29, 2018 – December 2, 2018)

Figure 2-14. Storm Event at San Jacinto River at Goetz Road (December 6-7, 2018)

Figure 2-15. Storm Event at San Jacinto River at Goetz Road (January 31, 2019 – February 4, 2019)

Event and annual mean concentrations for each analyte are presented in Table 2-6.  Event and annual loads for each analyte are presented in Table 2-7.  Concentrations for nutrients for the two storm events range from 1.3 to 2.0 mg/L for total nitrogen, and .045 to 0.80 mg/L for total phosphorus (Table 2-6).  Based on flow data provided by the nearby USGS stream gauge (Station ID 11070365), the total annual flow was estimated at 428,876,559 cf or 3,208.2 Mgal for the period of July 1, 2018 through June 30, 2019. No dry weather flows enter Canyon Lake from San Jacinto River at Goetz Road (Station ID 759) and the storm flows account for the estimated annual load of nutrients. The estimated annual nutrient load was calculated to be 20,457 kg for total nitrogen and 7,409 kg for total phosphorus (Table 2-7) for the period of July 1, 2018 through June 30, 2019.

Table 2-6. Water Quality Concentrations at San Jacinto River at Goetz Road

| Analyte | Units | Event 1 | Event 2 | Event 3 | Annual Mean | Annual Geomean |
| --- | --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | mg/L | 0.11 | 0.11 | 0.38 | 0.200 | 0.166 |
| Chemical Oxygen Demand | mg/L | 44 | 41 | 16 | 34 | 31 |
| Kjeldahl Nitrogen | mg/L | 1.0 | 1.2 | 0.89 | 1.0 | 1.0 |
| Nitrate as N | mg/L | 0.73 | 0.73 | 0.44 | 0.63 | 0.62 |
| Nitrite as N | mg/L | (0.099)J | (0.081)J | ND(<0.059) | 0.158 | 0.133 |
| Organic Nitrogen | mg/L | 0.9 | 1.1 | 0.51 | 0.8 | 0.8 |
| Total Nitrogen | mg/L | 1.8 | 2.0 | 1.33 | 1.7 | 1.7 |
| Total Phosphorus | mg/L | 0.45 | 0.80 | 0.57 | 0.61 | 0.59 |
| Ortho Phosphate Phosphorus | mg/L | 0.28 | 0.26 | 0.21 | 0.25 | 0.25 |
| Total Dissolved Solids | mg/L | 150 | 150 | 170 | 157 | 156 |
| Total Hardness | mg/L | 72 | 99 | 72 | 81 | 80 |
| Total Suspended Solids | mg/L | 32 | 360 | 150 | 181 | 120 |

Table 2-7. Water Quality Event and Annual Loads at San Jacinto River at Goetz Road

| Analyte | Units | Load Event 1 | Load Event 2 | Load Event 3 | Annual Load |
| --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | kg | 40 | 88 | 573 | 2,596 |
| Chemical Oxygen Demand | kg | 16,030 | 32,635 | 24,134 | 391,816 |
| Kjeldahl Nitrogen | kg | 364 | 955 | 1,342 | 12,422 |
| Nitrate as N | kg | 266 | 581 | 664 | 7,512 |
| Nitrite as N | kg | 36 | 64 | 445 | 2,046 |
| Organic Nitrogen | kg | 328 | 876 | 769 | 9,901 |
| Total Nitrogen | kg | 656 | 1,592 | 2,006 | 20,457 |
| Total Phosphorus | kg | 164 | 637 | 860 | 7,409 |
| Ortho Phosphate Phosphorus | kg | 102 | 207 | 317 | 2,995 |
| Total Dissolved Solids | kg | 54,648 | 119,396 | 256,428 | 1,915,004 |
| Total Hardness | kg | 26,231 | 78,801 | 108,605 | 981,172 |
| Total Suspended Solids | kg | 11,658 | 286,549 | 226,260 | 2,236,418 |

Hydrographs with flow-weighted sample aliquot times is provided in Figure 2-16, Figure 2-17, and Figure 2-18.  The figure was developed based on flow data provided by the nearby USGS stream gauge (Station ID 11070365).

Figure 2-16. Hydrograph of First Storm Event at San Jacinto River at Goetz Road 
(November 29, 2018 – December 2, 2018)

Figure 2-17. Hydrograph of Second Storm Event at San Jacinto River at Goetz Road 
(December 6-7, 2018)

Figure 2-18. Hydrograph of Third Storm Event at San Jacinto River at Goetz Road
(January 31, 2019 – February 4, 2019)

### Summary of Monitoring Data – San Jacinto River at Ramona Expressway

Mystic Lake did not overflow during the wet weather monitoring period from October 1, 2018 to May 31, 2019. Therefore, no samples were collected from the sampling station at San Jacinto River at Ramona Expressway (Station ID 741) during the 2018-2019 monitoring year. Flows from the local area were observed at the San Jacinto River at Ramona Expressway (Station ID 741) beginning on February 14, 2019, however, no flows originating from the upper watershed were observed and no flows exited Mystic Lake.

### Summary of Monitoring Data – Cranston Guard Station

The Cranston Guard Station site on the San Jacinto River was only monitored between 2007 and 2011 by the Forest Service. This work, however, was dependent on sufficient funds being allocated by Congress to complete the work.  In 2012, the Forest Service pulled out of the Task Force and no longer provides monitoring. Thus, no samples were collected during the 2018-2019 monitoring year.

### Summary of Monitoring Data – Canyon Lake Spillway

Water quality samples were collected during two storm events at Canyon Lake Spillway (Station ID 841) during the wet weather monitoring period from October 1, 2018 to May 31, 2019.

During the storm event on November 29, 2018 through December 2, 2018 conducted at Salt Creek at Murrieta Road (Station ID 745) and San Jacinto River at Goetz Road (Station ID 759), Canyon Lake Dam did not overflow, and no samples were collected at Canyon Lake Spillway (Station ID 841).

During the storm event on December 6, 2018 through December 7, 2018, conducted at Salt Creek at Murrieta Road (Station ID 745) and San Jacinto River at Goetz Road (Station ID 759), Canyon Lake Dam did not overflow, and no samples were collected at Canyon Lake Spillway (Station ID 841).

During the storm event on January 16, 2019 through January 20, 2019, a total of 46 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070500), flow for the storm event was estimated at 105,919,560 cf or 792.3 Mgal.

During the storm event on January 31, 2019 through February 6, 2019, a total of 49 discrete samples were collected across the hydrograph at two-hour intervals for the first 7 discrete samples and at three-hour intervals for the final 42 discrete samples. A single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070500), flow for the storm event was estimated at 138,921,444 cf or 1,039.2 Mgal.

During the storm event on February 14, 2019 through February 18, 2019, a total of 35 discrete samples were collected across the hydrograph at three-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070500), flow for the storm event was estimated at 438,598,278 cf or 3,281.9 Mgal.

Photos taken during the storm events are provided in Figure 2-19, Figure 2-20, and Figure 2-21.

Figure 2-19. Storm Event at Canyon Lake Spillway (January 16-20, 2019)

Figure 2-20. Storm Event at Canyon Lake Spillway (January 31, 2019 – February 6, 2019)

Figure 2-21. Storm Event at Canyon Lake Spillway (February 14, 2019 – February 18, 2019)

Data for the EMCs of each analyte are presented in Table 2-8. Event and annual loads for each analyte are presented in Table 2-9. Concentrations for nutrients for the two storm events range from 1.1 to 1.6 mg/L for total nitrogen, and 0.11 to 0.32 mg/L for total phosphorus (Table 2-8). Based on flow data provided by the nearby USGS stream gauge (Station ID 11070365), the total annual flow was estimated at 787,726,440 cf or 5,892.6 Mgal for the period of July 1, 2018 through June 30, 2019. The USGS stream gauge (Station ID 11070365) located downstream of the Canyon Lake Spillway (Station ID 841) sampling location has minimal dry weather flow and storm flows account for the vast majority of the estimated annual load of nutrients exiting Canyon Lake. The estimated annual nutrient load was calculated to be 32,832 kg for total nitrogen and 5,416 kg for total phosphorus (Table 2-9) for the period of July 1, 2018 through June 30, 2019.

Table 2-8. Water Quality EMCs at Canyon Lake Spillway

| Analyte | Units | Event 1 | Event 2 | Event 3 | Annual Mean | Annual Geomean |
| --- | --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | mg/L | 0.21 | (0.070)J | 0.11 | 0.130 | 0.117 |
| Chemical Oxygen Demand | mg/L | 35 | ND(<7.4) | 41 | 19 | 11 |
| Kjeldahl Nitrogen | mg/L | 1.3 | 0.86 | 1.1 | 1.1 | 1.1 |
| Nitrate as N | mg/L | (0.17)J | 0.21 | 0.48 | 0.29 | 0.26 |
| Nitrite as N | mg/L | ND(<0.059) | ND(<0.059) | 0.069 | 0.220 | 0.182 |
| Organic Nitrogen | mg/L | 1.1 | 0.8 | 0.96 | 1.0 | 0.9 |
| Total Nitrogen | mg/L | 1.5 | 1.1 | 1.6 | 1.4 | 1.4 |
| Total Phosphorus | mg/L | 0.11 | 0.14 | 0.32 | 0.19 | 0.17 |
| Ortho Phosphate Phosphorus | mg/L | ND(<0.016) | ND(<0.016) | 0.094 | 0.04 | 0.02 |
| Total Dissolved Solids | mg/L | 510 | 470 | 430 | 470 | 469 |
| Total Hardness | mg/L | 240 | 230 | 220 | 230 | 230 |
| Total Suspended Solids | mg/L | 14 | 16 | 150 | 60 | 32 |

Table 2-9. Water Quality Event and Annual Loads at Canyon Lake Spillway

| Analyte | Units | Load Event 1 | Load Event 2 | Load Event 3 | Annual Load |
| --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | kg | 630 | 275 | 1,366 | 2,655 |
| Chemical Oxygen Demand | kg | 104,976 | 14,555 | 509,209 | 685,882 |
| Kjeldahl Nitrogen | kg | 3,899 | 3,383 | 13,662 | 24,153 |
| Nitrate as N | kg | 510 | 826 | 5,961 | 8,144 |
| Nitrite as N | kg | 885 | 1,160 | 857 | 3,551 |
| Organic Nitrogen | kg | 3,299 | 3,147 | 11,923 | 21,184 |
| Total Nitrogen | kg | 4,499 | 4,327 | 19,872 | 32,832 |
| Total Phosphorus | kg | 330 | 551 | 3,974 | 5,416 |
| Ortho Phosphate Phosphorus | kg | 24 | 31 | 1,167 | 1,331 |
| Total Dissolved Solids | kg | 1,529,647 | 1,848,894 | 5,340,480 | 10,106,970 |
| Total Hardness | kg | 719,834 | 904,778 | 2,732,338 | 5,036,159 |
| Total Suspended Solids | kg | 41,990 | 62,941 | 1,862,958 | 2,145,074 |

Hydrographs with flow-weighted sample aliquot times is provided in Figure 2-22, Figure 2-23, and Figure 2-24.  The figure was developed based on flow data provided by the nearby USGS stream gauge (Station ID 11070365). A hydrograph of the Canyon Lake Level at Railroad Canyon Dam Spillway compared to the spillway elevation is provided in Figure 2-25.

Figure 2-22. Hydrograph of First Storm Event at Canyon Lake Spillway
(January 16-20, 2019)

Figure 2-23. Hydrograph of Second Storm Event at Canyon Lake Spillway
(January 31, 2019 – February 6, 2019)

Figure 2-24. Hydrograph of Third Storm Event at Canyon Lake Spillway
(February 14, 2019 – February 18, 2019)

Figure 2-25. Canyon Lake Level at Railroad Canyon Dam Spillway

## San Jacinto Watershed Rainfall Records

The RCFC&WCD maintains rainfall records for rain gauges located within or near the San Jacinto Watershed as shown in Table 2-8.

Table 2-10. San Jacinto River Watershed Rainfall Gauges

| Station ID | Station Description | Latitude | Longitude | Elevation (ft.) |
| --- | --- | --- | --- | --- |
| 67 | Lake Elsinore | 33.668712 | -117.332380 | 1281 |
| 152 | Perris | 33.786980 | -117.231831 | 1494 |
| 155 | Perris / Moreno Valley – Pigeon Pass | 33.987703 | -117.270221 | 1902 |
| 186 | Hemet / San Jacinto | 33.787067 | -116.959024 | 1554 |
| 248 | Winchester | 33.702903 | -117.090382 | 1466 |

Rainfall data recorded at these five stations for the period July 1, 2018, through June 30, 2019, are summarized in Table 2-9.  The complete set of rainfall gauge data is included in Appendix A.

| Table 2-11. Summary Rainfall Data (July 2018 to June 2019) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Monthly Rainfall (inches) | Lake Elsinore | Perris CDF | Pigeon Pass | Hemet / San Jacinto | Winchester |
| Jul | 0.00 | 0.00 | 0.07 | 0.05 | 0.02 |
| Aug | 0.05 | 0.00 | 0.06 | 0.00 | 0.00 |
| Sep | 0.02 | 0.00 | 0.00 | 0.00 | 0.00 |
| Oct | 1.33 | 0.79 | 0.93 | 0.46 | 0.69 |
| Nov | 0.62 | 1.16 | 1.26 | 1.30 | 1.07 |
| Dec | 1.57 | 1.35 | 1.27 | 1.57 | 1.22 |
| Jan | 2.50 | 2.45 | 3.20 | 2.49 | 2.03 |
| Feb | 6.02 | 5.16 | 7.19 | 6.75 | 5.38 |
| Mar | 1.59 | 1.42 | 2.74 | 1.64 | 2.20 |
| Apr | 0.00 | 0.00 | 0.06 | 0.40 | 0.00 |
| May | 0.68 | 1.05 | 1.72 | 1.61 | 0.76 |
| Jun | 0.00 | 0.00 | 0.02 | 0.02 | 0.00 |
| Annual Rainfall (Inches) | 14.38 | 13.38 | 18.52 | 16.29 | 13.37 |

# In-Lake Monitoring

## Background

Routine in-lake monitoring was initiated in 2006 by local stakeholders in cooperation with the RWQCB at three open water locations in Lake Elsinore and four locations in Canyon Lake.  Previous monitoring consisted of monthly sampling October to May, and biweekly sampling June to September, with grab samples collected at the surface, within the water column, and/or as depth-integrated samples (depending on the lake and the analyte).  Based on modifications adopted to the sampling program (Regional Board Resolution No. R8-2011-0023), in 2011-2012 sampling locations in Lake Elsinore and Canyon Lake were reduced to one and three stations, respectively for analytical chemistry. This decision was based on a review of available data that indicated consistent similar nutrient concentrations and physical water quality parameters among the three sampling sites in Lake Elsinore and two sites in the eastern arm of Canyon Lake.  This savings also shifted resources toward several implementation strategies aimed at reducing nutrient impacts in both lakes as described in RWQCB Resolution No. R8-2011-0023.  All in-lake monitoring was then suspended temporarily during the 2013-2014 and 2014-2015 FYs to further redirect additional resources toward implementing in-lake best management practices.  However, ongoing in-lake sampling has resumed and is required to estimate progress toward attaining nutrient TMDL targets and calculating annual and 10-year running averages. The following sections describe monitoring methods and results in both lakes for the 2018-2019 FY.

Of note during this monitoring year was a large wildfire (the Holy Fire) that occurred just to the north of Lake Elsinore.  The Holy Fire began on August 6, 2018 and reached full containment on September 13, 2018 with a total burned area of 23,025 acres (35.9 square miles).  Lake Elsinore is within the Holy Fire burned watershed area, with McVicker, Leach and Dickey Canyons draining to the north end of the lake. Since containment of the fire, the area has received numerous storms of varying intensity, totaling approximately 20 inches of rain from October 4, 2018 through February 14, 2019, resulting in debris flows from these catchment areas into Lake Elsinore and creating a large sediment plume at the mouth of the Leach Canyon Channel. Approximately two weeks after an early December 2018 storm dropped 2 inches of rain in the burn area, a large fish die-off (primarily common carp and threadfin shad) was observed in Lake Elsinore which continued through January 2019. Based on multiple lines of evidence this die-off was attributed to the golden algae, Prymnesium parvum, a species not previously observed at high densities in the lake. Five phytoplankton samples collected on January 9, 2019 at various locations around the lake confirmed that golden algae were present in all samples at densities high enough to cause or contribute to a fish die-off. Dissolved oxygen concentrations remained > 6.0 milligrams (mg) per liter in all areas of the lake monitored as confirmed by January 9 water column profiles and in-situ continuous data sondes.

## Lake Elsinore Monitoring

### Sampling Station Locations and Frequency

To maintain consistency and facilitate the assessment of trends toward meeting compliance goals, the in-lake monitoring design was resumed using the three former stations outlined in the approved Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan (LESJWA, 2006; Figure 3-1, Table 3-1).  Analytical chemistry samples and in-situ water quality profile readings were collected at Site LE02, while only in-situ water quality profile readings were performed at the remaining two stations (LE01 and LE03). Profile readings for all three stations were taken in both the morning and afternoon. Water chemistry samples collected at Site LE02 were analyzed for those constituents outlined in Table 3-2.  Sampling in Lake Elsinore was conducted monthly during summer months (June-September) and bi-monthly (i.e. every other month) for the remainder of the monitoring year. In-lake sampling events were coordinated to correspond with satellite overpass dates to facilitate the comparison of in-lake and satellite derived chlorophyll-a data (see Section 3.4).

Table 3-1.  Lake Elsinore Monitoring Stations

| Site | Latitude | Longitude |
| --- | --- | --- |
| LE01 | 33.668978° | -117.364185° |
| LE02 | 33.663344° | -117.354213° |
| LE03 | 33.654939° | -117.341653° |

Figure 3-1. Lake Elsinore Sampling Locations

Table 3-2.  2018-2019 In-lake Analytical Constituents and Methods for Lake Elsinore

| Parameter | Analysis Method | Sampling Method |
| --- | --- | --- |
| Analytical Chemistry |  |  |
| Nitrite Nitrogen (NO2-N) | EPA 300.0 | Depth Integrated |
| Nitrate Nitrogen (NO3-N) | EPA 300.0 | Depth Integrated |
| Total Kjeldahl Nitrogen (TKN) | EPA 351.3 | Depth Integrated |
| Total Nitrogen (TN) | Calculated | NA |
| Ammonia Nitrogen (NH4-N) | SM4500-NH3 H | Depth Integrated |
| Sulfide | SM 4500S2 D | Depth Integrated |
| Total Phosphorus (TP) | EPA 365.1 | Depth Integrated |
| Soluble Reactive Phosphorus (SRP / Ortho-P) | SM 4500-P E | Depth Integrated |
| Chlorophyll-a | SM 10200H | Surface (0-2m) & Depth Integrated |
| Total Dissolved Solids (TDS) | SM 2540 C | Depth Integrated |

NA – not applicable

### Sampling Methods

Depth-integrated composite samples for analytical chemistry were collected at Site LE02 by utilizing a peristaltic pump and lowering/raising an inlet tube through the water column at a uniform speed, creating a composite sample of the entire water column. Two discrete samples were collected for chlorophyll-a: 1) a full depth-integrated composite sample as described above; and 2) a 0-2-meter (m) depth-integrated composite surface sample. All samples for chemical analysis were placed and held on wet ice immediately following collection and transferred to a local courier or shipping company on the same day of collection. Samples for analysis of nutrients, ammonia, sulfide, TDS, and chlorophyll-a were submitted to Babcock Laboratories Inc. located in Riverside, California.

Secchi disk readings for water clarity, as well as in-situ water column profile data and were recorded in the morning at all three Lake Elsinore stations using pre-calibrated hand-held YSI field meters or equivalent for pH, temperature, DO, and specific conductivity at 1-m intervals throughout the water column.  This data was used to assess lateral and vertical spatial variability within the lake.  End-of-the-day water column profiles (i.e. after ~2:00pm) were also recorded for the same suite of parameters at all three stations to assess any potential temporal variability in these parameters over the course of a day.

Satellite imagery was used as a tool to remotely measure chlorophyll-a and turbidity concentrations.  These images provide a more complete picture of spatial variability that can exist for these two parameters at any given point in time. In-lake sampling dates were selected to correspond with satellite overpasses to enable comparison of analytical laboratory and satellite derived chlorophyll-a concentrations.  Processed satellite imagery and associated reports were provided by EOMAP GmbH & co. KG (EOMAP) based in Germany (Castle Seefeld Schlosshof).  Satellite imagery was also used to estimate the likelihood of a harmful algae bloom.

### Water Quality Summary

A summary of the in-lake monitoring events for Lake Elsinore for the period of July 23, 2018 to June 24, 2019 is presented below.  A total of eight Lake Elsinore events were sampled during this period under the TMDL monitoring program, with five occurring in 2018 (July 23, August 24, September 11, October 16 and December 20) and three in 2019 (February 19, April 9 and June 24).  Complete water quality profile measurements can be found in the quarterly reports contained in Appendix B.  Detailed analytical chemistry lab reports for each event are contained in Appendix C.  Satellite imagery reports for each event are provided in Appendix D.

A summary of mean water column profile values for each site and monitoring event are presented in Tables 3-3 and 3-4.  Water column mean profile statistics for each site across the entire monitoring period are presented in Table 3-5.  Mean values for water column measurements are also summarized graphically in Figures 3-2 through 3-5 for each site.  The measurements during the morning and afternoon of any given monitoring event were averaged prior to summarizing in the tables and figures below.

Table 3-3. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2018-2019 Monthly Means for Each Site (July – Dec 2018)

Notes:

°C = degrees Celsius; µS/cm = microsiemens per centimeter; mg/L = milligrams per liter.

Table 3-4. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2018-2019 Monthly Means for Each Site (Feb – June 2019)

Notes:

°C = degrees Celsius; µS/cm = microsiemens per centimeter; mg/L = milligrams per liter.

Temperature exhibited a typical pattern with lowest values occurring during the winter events (December and February) and highest values in summer months (July and August).  The greatest mean DO concentrations throughout the water column were observed in December at Sites LE01, LE02 and LE03. Conductivity increased from July through October 2018, from approximately 4800 to 5200 microsiemens per centimeter (µS/cm). The conductivity dropped slightly between October 2018 and December 2018, due to the occurrence of two storms dropping approximately 3.5 inches of rain. Conductivity then declined sharply between December 2018 and February 2019 after several large storms.  The conductivity then remained relatively stable through June 2019. The overall annual average pH in the lake was 8.93. The pH values remained within a relatively narrow band across the monitoring year, only varying by approximately 0.35 units.

Concentrations of DO near the bottom tracked closely with the overall water column mean for all three sites.  The water column mean DO concentration was above the 2015 TMDL target of 5.0 mg/L as a rolling average for all events at Site LE02 (Figure 3-2).  However, the rolling average DO concentration was below the 2020 TMDL target of 5.0 mg/L 1-m from the lake bottom for all 2018-19 monitoring events among at Site LE02 (Figure 3-2).

Secchi depths at Site LE02 decreased across the summer and fall months from 0.25-m to 0.15-m, then gradually increased across the winter months, reaching their highest values at the end of the monitoring period at 0.4-m (Figure 3-4b). The Secchi depths observed generally exhibit the same pattern as analytical chlorophyll-a concentrations (i.e. algal density) measured.  Sites LE01 and LE03 both displayed similar Secchi depths and patterns.

Table 3-5. In-Situ Water Quality Parameter Measurements in Lake Elsinore - Annual Mean Statistics for Each Site

Notes:

°C = degrees Celsius; µS/cm = microsiemens per centimeter; mg/L = milligrams per liter.

Figure 3-2. Water Column Mean Dissolved Oxygen (DO) Rolling Average – Lake Elsinore – Site LE02

Values are calculated by averaging the measurement from each event with the previous seven events (i.e. one year of data) to obtain a rolling average.

Figure 3-3. In-Situ Physical Water Quality Parameters - Lake Elsinore - Site LE01

a - TMDL Target for dissolved oxygen is depth average no less than 5 mg/L no later than 2015, no less than 5 mg/L 1-m above lake bottom no later than 2020.

Figure 3-4a. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE02

a-TMDL Target for dissolved oxygen is depth average no less than 5 mg/L no later than 2015, no less than 5 mg/L 1-m above lake bottom no later than 2020

Figure 3-4b. In-Situ Secchi Depth Measurements - Lake Elsinore Site LE02

Figure 3-5. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE03

a-TMDL Target for dissolved oxygen is depth average no less than 5 mg/L no later than 2015, no less than 5 mg/L 1-m above lake bottom no later than 2020

#### Analytical Chemistry

Monthly analytical results and annual summaries chemistry concentrations at Site LE02 are presented in Tables 3-6 and 3-7, respectively.  Concentrations of analytes at Site LE02 are graphically presented in Figures 3-6 through 3-9.

Total nitrogen levels were higher during the first half of the monitoring period, before decreasing across the winter months and into the second half of the monitoring year, likely as a result of the winter rains. The maximum value of total nitrogen was measured at 6.9 mg/L in September 2018 and the minimum total nitrogen value was measured at 2.8 mg/L in April 2018. The annual mean concentration of total nitrogen was 5.0 mg/L (5.1 mg/L in the previous 2017-2018 monitoring year). The total nitrogen rolling average concentration exceeded the current 2020 TMDL target of 0.75 mg/L for each event (Figure 3-7).

Total phosphorus concentrations ranged from 0.02 to 0.23 mg/L across all monitoring events.  Some unusually sharp decreases in total phosphorus were noted in both September 2018 and February 2019.  The decrease in total phosphorus in February is likely the result of the influx of water from the large rain events and Canyon Lake dam overflow.  While the decrease in September could be due to scavenging of phosphorous by phytoplankton during blooms observed in July and August, this pattern has not been observed during prior summers.   The annual mean concentration of total phosphorus was 0.15 mg/L, which is very similar to the 0.14 mg/L annual mean from the previous monitoring year.  The total phosphorus rolling average concentration exceeded the current 2020 TMDL target of 0.1 mg/L for each event (Figure 3-7).

Total ammonia-N concentrations ranged from <0.048 to 0.24 mg/L.  The July 2018 sample concentration of 0.24 mg/L total ammonia-N exceeded the Criterion Continuous Concentration (CCC) value of 0.21 mg/L for the protection of aquatic life.  No other samples exceeded the CCC or the US EPA acute water quality Criterion Maximum Concentration (CMC) value (Table 3-7).

Total dissolved solids (TDS) concentration gradually increased between July and October 2018 (2900-3200 mg/L), before gradually declining for the remainder of the monitoring period down to 2000 mg/L).  For all events, the TDS remained at or above the Basin Plan Objective of 2000 mg/L, with an annual mean concentration of 2675 mg/L.

Chlorophyll-a depth-integrated concentrations across all eight sampling events ranged from 66.5 to 144 µg/L.  Surface (0-2m) chlorophyll-a concentrations ranged from 55 to 154 µg/L. Depth-integrated chlorophyll-a concentrations exhibit a general downwards trend across the monitoring year, with a slight increase in December 2018.  The mean chlorophyll-a concentration observed in samples collected during the summer months (June 2018 through September 2018) was 105 µg/L for depth-integrated samples and 119 µg/L for surface samples.  These concentrations have declined from the previous monitoring year, where mean summer concentrations were 149 µg/L for depth-integrated samples and 161 µg/L for surface samples (Wood 2018).

The current 2018-2019 FY Lake Elsinore data in the context of historical data can be found in Appendix E.

Table 3-6.  Monthly Analytical Chemistry Results for Lake Elsinore in 2018-2019

Notes:

a - Total Nitrogen = TKN+NO2+NO3

b - Values are site specific dependent upon pH and temperature recorded at each location. Calculated using equation by Thursby (1986).

c – Un-ionized ammonia concentrations calculated based on ½ detection limit when ammonia nitrogen was ND

NS – Not sampled; ND – Not detected; NA – Not Applicable/ available

DI = Depth integrated; SG = Surface grab; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; m – meters; RL – reporting limit; J- Reported value was detected above the Method Detection Limit (MDL), but below the Reporting Limit (RL)

Table 3-7. Analytical Chemistry Summary for Lake Elsinore – Annual Mean Statistics (2018-2019)

Notes:

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Summer average

d - Values calculated using water column mean ammonia, temperature, salinity and pH.  Calculated using equation by Thursby (1986).

e - Values are the mean of June through September 2018 results

f – Un-ionized ammonia concentrations calculated based on ½ detection limit when ammonia nitrogen was ND

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

NA – Not applicable/ available

DI = Depth integrated; Surf = Surface 0-2m

mg/L – micrograms per liter; ug/L – milligrams per liter; m – meters; RL – reporting limit

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

.

Figure 3-6. Lake Elsinore Analytical Chemistry – Depth-Integrated Means 
(July 2018-June 2019)

Figure 3-7. Lake Elsinore Analytical Chemistry – Rolling Averages 
(July 2018 – June 2019)

Values are calculated by averaging the value of each event with the previous seven events (i.e. one year of data) to obtain a rolling average.

Figure 3-8. Lake Elsinore Analytical Chemistry – Depth-Integrated and Surface Chlorophyll-a

## Canyon Lake Monitoring

### Sampling Station Locations and Frequency

Similar to Lake Elsinore, sampling parameters and locations in Canyon Lake were based on the TMDL monitoring conducted between 2006 and 2012 to provide consistency in assessing trends toward meeting compliance goals.  The in-lake monitoring design halted in 2012 resumed using the four stations outlined in the approved Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan (LESJWA, 2006; Figure 3-9, Table 3-8). Two sites are located in the main body of the lake (CL07 near the dam and CL08 in the northern arm), and two in the East Bay (CL09 and CL10).  Samples for analytical chemistry and chlorophyll-a, were collected at all four sites, in addition to morning and afternoon in-situ water quality profile readings.

Sampling in Canyon Lake was conducted bi-monthly (i.e., every other month) concurrent to the TMDL sampling in Lake Elsinore, and also coordinated with satellite overpass dates (see Section 3.4).

Table 3-8.  Canyon Lake Sampling Station Locations

| Site | Latitude | Longitude |
| --- | --- | --- |
| CL07 | 33.678027° | -117.275135° |
| CL08 | 33.688211° | -117.268944° |
| CL09 | 33.681100° | -117.258892° |
| CL10 | 33.679495° | -117.250669° |

Figure 3-9. Canyon Lake Sampling Locations

### Sampling Methods

Samples for analytical chemistry were collected in the same manner as in Lake Elsinore using a peristaltic pump.  Two discrete samples were collected for chlorophyll-a: 1) a full depth-integrated composite sample; and 2) a 0-2-m depth-integrated composite surface sample. All analytical samples were held on wet ice immediately following collection and transferred to a local courier or shipping company on the same day of collection. Samples for analysis of nutrients, ammonia, sulfide, TDS, and chlorophyll-a were submitted to Babcock Laboratories Inc. located in Riverside, California (Table 3-9).

Beginning with the February 2017 sampling event, the TMDL Task Force directed that the pre- and post-alum application monitoring be integrated into the routine TMDL monitoring, given that the monitored analytes were largely identical to the TMDL monitoring, with the exception of aluminum and total suspended solids.  Given this directive, total/dissolved aluminum and total suspended solids were added to the nutrient TMDL monitoring analyte list for all subsequent routine TMDL monitoring events on Canyon Lake. During the 2018-2019 monitoring period Canyon Lake alum applications were performed during the week of September 17, 2018 and again during the week of April 15, 2019.  Pre-alum application monitoring events were performed on September 11, 2018 and April 9, 2019, with the subsequent respective bi-monthly TMDL event serving as the post-alum application monitoring.

In-situ water column profile data was recorded in the morning at all four Canyon Lake stations using pre-calibrated hand-held YSI field meters or equivalent for pH, temperature, DO, and specific conductivity at 1-m intervals throughout the water column.  These data were used to assess lateral and vertical spatial variability within the lake.  End-of-the-day water column profiles (i.e. after ~2:00pm) were also recorded for the same suite of parameters at all stations to assess any potential temporal variability in these parameters over the course of a day.  Water clarity was also assessed with a Secchi disk at all stations.

Satellite imagery was used to remotely measure chlorophyll-a and turbidity concentrations in Canyon Lake.  Satellite imagery was also used to estimate the likelihood of a harmful algae bloom.

Table 3-9.   In-lake Analytical Constituents and Methods for Canyon Lake (2018-2019)

| Parameter | Analysis SOP # | Sampling Method |
| --- | --- | --- |
| Analytical Chemistry |  |  |
| Nitrite Nitrogen (NO2-N) | SM4500-NO2 B | Depth Integrated |
| Nitrate Nitrogen (NO3-N) | EPA 300.0 | Depth Integrated |
| Total Kjeldahl Nitrogen (TKN) | EPA 351.3 | Depth Integrated |
| Total Nitrogen (TN) | Calculated | NA |
| Ammonia Nitrogen (NH4-N) | SM4500-NH3 H | Depth Integrated |
| Sulfide | SM 4500S2 D | Depth Integrated |
| Total Phosphorus (TP) | SM4500-P E | Depth Integrated |
| Soluble Reactive Phosphorus (SRP / Ortho-P) | SM4500-P E | Depth Integrated |
| Chlorophyll-a | SM 10200H | Surface (0-2m) & Depth Integrated |
| Total Dissolved Solids (TDS) | SM 2540 C | Depth Integrated |
| Total Suspended Solids (TSS) | SM 2540D | Depth Integrated |
| Total Aluminum | EPA 200.7 | Depth Integrated |
| Dissolved Aluminum | EPA 200.7 | Depth Integrated |

Notes

NA – not applicable

### Water Quality Summary

A summary of the in-lake monitoring events for Canyon Lake for the period of July 1, 2018 to June 30, 2019 is presented below.  A total of six events were sampled under the TMDL monitoring program, with three occurring in 2018 (August 24, October 16, December 20) and three in 2018 (February 19, April 9, and June 24).  Complete water quality profile measurements can be found in the quarterly reports contained in Appendix B.  Detailed analytical chemistry lab reports for each event are contained in Appendix C.  Satellite imagery reports for each event are provided in Appendix D.

#### Water Column Profiles

A summary of water column profile mean values for each site and monitoring event are presented in Tables 3-10 and 3-11.  A summary of water column profile mean values for each basin (i.e. Main Lake and Eastern Arm) are presented in Tables 3-12 and 3-13. Water column profile mean statistics for each site across the entire monitoring period are presented in Table 3-14.  Mean water column values across the annual cycle are summarized graphically in Figures 3-10a and 3-10b.

For the purposes of this report, the epilimnion is defined as the region of the water column above the thermocline, while the hypolimnion is the region of the water column below the thermocline, with both regions exhibiting relatively stable temperatures.  The thermocline portion of the water column is defined as the region between the epilimnion and hypolimnion where a marked drop in temperature per unit of depth is evident. Measurements within the thermocline were excluded from epilimnion and hypolimnion averaging. Full water column means included data recorded from all three zones, if present.

For both the Main Basin and East Basin, temperature exhibited a typical pattern with lowest values occurring during the winter events (February) and highest values in summer months (August).  Dissolved oxygen concentrations for both basins reflected the same spike in the winter months when averaged throughout the water column, reaching their highest concentrations in December 2018 (East Basin) and February 2019 (Main Basin). When the thermocline developed, DO concentrations within the epilimnion and hypolimnion diverged, with hypolimnion concentrations falling substantially during that timeframe to <0.5 mg/L (Figures 3-10a and 3-10b).  The rolling annual average DO concentration within the epilimnion was greater than the current 2015 TMDL target of 5.0 mg/L for all five events in both Basins when stratification was present (Figure 3-11).  However, the rolling average DO concentration was never above the target of 5.0 mg/L in the hypolimnion when stratification was present.  The rolling average of the full water column mean was above 5.0 mg/L for all monitoring dates (Figure 3-12).

Conductivity within the epilimnion and hypolimnion (when present) remained consistent throughout the monitoring period.  Average specific conductivity throughout the entire water column in the Main Basin of Canyon Lake (mean of CL 07 & CL 08) ranged from 516-948 µS/cm, remaining consistent across the summer and fall months, decreasing after winter storms, and then slowly increasing again near the end of the monitoring period (Tables 3-12 and 3-13).  Locations in the East Basin (mean of CL09 & CL10) exhibited a similar pattern with mean water column values across the monitoring year ranging from 521-1186 µS/cm Mean values for pH were slightly higher in the Eastern Basin than the Main Basin, with values ranging from 7.40 – 8.51 and 7.40 – 7.87, respectively.  Values for pH within the epilimnion and hypolimnion tended to diverge as the thermocline developed.

Secchi depths were generally limited to a narrow range for both basins, with the exceptions of the February and April 2019 measurements (Figure 3-13).  Following the large February 2019 storms the Secchi depths exhibited a notable decrease, this was followed by a large increase in Secchi depths during the April 2019 monitoring effort. During the first three monitoring events (August – December) Secchi depths ranged from 1.2 to 1.6-m in the Main Basin (mean of CL07 & CL08) but decreased to 0.18-m in February and then increased to 4.3-m in April 2019.  The East Basin (mean of CL09 & CL10) showed a similar trend to a somewhat lesser degree, with Secchi depths ranging from 0.73 to 1.0-m from August to December 2018, decreasing to 0.1-m in February 2019, and then increasing to 2.0-m in April 2019.  This may be attributed to the series of large storm events in February 2019, bringing in large volumes of water with high concentrations of suspended sediments causing decreased water clarity (see Figure 3-14).  The suspended material settled resulting in much clearer water in April 2019.

Table 3-10. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (Aug – Dec 2018)

-- = not applicable due to lack of thermocline

Table 3-11. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (Feb – June 2019)

-- = not applicable due to lack of thermocline

Table 3-12. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (Aug – Dec 2018)

-- = not applicable due to lack of thermocline

Table 3-13. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (Feb – June 2019)

-- = not applicable due to lack of thermocline

Table 3-14. In-Situ Water Quality Parameter Measurements for Canyon Lake - Annual Mean Statistics for Each Site (Aug 2018 – June 2019)

NA – not applicable due to lack of thermocline

Values for epilimnion and hypolimnion when present

Figure 3-10a. Mean In-Situ Physical Water Quality Parameters – Canyon Lake Main Basin

a - TMDL Target for dissolved oxygen is no less than 5 mg/L above thermocline no later than 2015, no less than 5 mg/L in hypolimnion no later than 2020

(Values represent the mean of Sites CL07 & CL08. Missing values represent time periods when no stratification was present)

Figure 3-10b. Mean In-Situ Physical Water Quality Parameters - Canyon Lake East Basin

a -TMDL Target for dissolved oxygen is no less than 5 mg/L above thermocline no later than 2015, no less than 5 mg/L in hypolimnion no later than 2020

(Values represent the mean of Sites CL09 & CL10.  Missing values represent time periods when no stratification was present.)

Figure 3-11. Rolling Average Concentrations of Dissolved Oxygen in the Epilimnion and Hypolimnion of Canyon Lake

Values are calculated by averaging the value from each event with the previous five event values (i.e. one year of data) to obtain a rolling average.

Figure 3-12. Rolling Average Concentration of Dissolved Oxygen Across the Full Vertical Water Column in Canyon Lake

Values are calculated by averaging the value from each event with the previous five event values (i.e. one year of data) to obtain a rolling average.

Figure 3-13. In- Situ Secchi Depth Measurements – Main and East Basins

Figure 3-14. Canyon Lake during February 2019 Monitoring Event Following a Series of Large Storm Events in January and February.  Note highly turbid water.

#### Analytical Chemistry

Summaries of analytical chemistry concentrations for each monitoring event in Canyon Lake are presented in Tables 3-15 and 3-16. A summary of analytical chemistry mean statistics for each site across the entire monitoring period are presented in Tables 3-17 through 3-19. Concentrations of analytes are presented graphically in Figures 3-15 and 3-16.  The data displayed in these figures shows average concentrations for the two locations in the Main Basin and the two locations in the East Basin during each sampling event.  The results discussed below include the full range of vales measured at all four locations.

Total nitrogen concentrations in the Main Basin (at sites CL07 and CL08) ranged from 0.90 to 3.26 mg/L across the six sampling events, with an annual mean of 1.54 mg/L (up from the 2017-18 annual mean of 1.20 mg/L).  Total nitrogen concentrations at the two East Basin sites ranged from 0.80 to 2.20 mg/L across the six sampling events, with the annual mean of 1.43 mg/L (up from the 2017-18 annual mean of 1.17 mg/L). The rolling average for total nitrogen \ exceeded the current 2020 TMDL target of 0.75 mg/L for all sites during all events (Figure 3-15).

Total phosphorus concentrations among the two locations in the Main Basin across the monitoring year ranged from 0.02 to 0.27 mg/L, with an annual mean of 0.10 mg/L.  Total phosphorus concentrations in the East Basin ranged from 0.03 to 0.44 mg/L, with an annual mean of 0.13 mg/L.  This demonstrates an overall increase in total phosphorus in both basins from the previous monitoring year, with annual means of 0.09 and 0.06 mg/L for the Main Basin and East Basins, respectively.  The rolling average for total phosphorus across all sites was below the current 2020 TMDL target of 0.1 mg/L for the first four monitoring events, but then increased above the target during the last two events of the monitoring year (Figure 3-17).  An anomaly was observed with February 2019 ortho-phosphate phosphorus samples from all Canyon Lake samples, in that the ortho-phosphate phosphorus (i.e. dissolved phosphorus) was found in higher concentrations than the total phosphorus.  It is likely that some matrix interference influenced the results of these samples, as the water in Canyon Lake during this sampling event was extremely turbid from a large storm 5 days prior.  The concurrent ortho-phosphate phosphorus sample from Lake Elsinore, prepared and run in the same analytical laboratory batch did not exhibit this pattern, further indicating a Canyon Lake specific irregularity.

Depth-integrated samples in the Main Basin exhibited a steady decline in chlorophyll-a concentrations through April 2018, then increased slightly in June 2019. The East Basin exhibited more variable pattern with lower concentrations of chlorophyll-a in late winter and spring (February and April), and late summer (August), and greater concentrations in December and June. Depth-integrated concentrations in the Main Basin (mean of Sites CL07 and CL08) across all six sampling events ranged from 1.0 to 23 µg/L, with a mean of 13 µg/L.  Depth-integrated concentrations of chlorophyll-a in the East Basin (Sites CL09 and CL10) across all events ranged from 2.0 to 49 µg/L, with a mean of 16 µg/L.  The lake-wide chlorophyll-a rolling average was slightly above the 2020 TMDL target of 25 µg/L for the first sampling event, but decreased and was below the target for the remainder of the events (Figure 3-17).

Total ammonia concentrations observed in the East Basin showed a gradual increase in values throughout the monitoring period, ranging from non-detect (<0.048) to 1.3 mg/L among the two East Basin sites, with an annual mean of 0.35 mg/L. While the annual mean of all measurements in the Main Basin was similar at 0.57 mg/L. A large spike in total ammonia was noted in the Main Basin in October 2018 up to 2.5 mg/L at Site CL07.  However, no individual samples exceeded the US EPA CMC or CCC values for the protection of aquatic life.

Total dissolved solids concentrations for both basins followed a very similar pattern over the course of the monitoring year. Both basins decreased gradually between August 2018 and February 2019, before slowly increasing again for the remainder of the monitoring period. The average TDS in the Main Basin steadily decreased from 610 mg/L in August 2018 to 345 mg/L in February 2019, and then steadily rose again to 430 mg/L at the end of the monitoring period. Similarly, the average concentrations of TDS in the East Basin decreased from 785 to 365 mg/L, and then increased to 495 mg/L. TDS concentrations in the East Basin exceeded the Basin Plan water quality objective of 700 mg/L twice (August and October of 2018).

Concentrations of total aluminum ranged from ND (< 37 µg/L) to 3,800 µg/L in the Main Basin and ND (< 37 µg/L) to 6,300 µg/L in the East Basin among all sampling locations and dates.  Generally, concentrations of total aluminum were consistent across the monitoring period (<210 µg/L). However, substantial spikes in total aluminum were observed in both basins in February 2019.  This was likely the result of the large influx of sediment-laden stormwater during January and February 2019.  A large portion of the earth’s crust is composed of aluminum, so this increase after large storms might be expected.  Dissolved aluminum was not detected at concentrations greater than the MDL in any sample, with the exception of October 2018 and June 2019 (35 to 61 µg/L). Detected concentrations of dissolved aluminum were all below the RL of 100 µg/L.

The current 2018-2019 FY Canyon Lake data in the context of historical data can be found in Appendix E.

Table 3-15. Analytical Chemistry Results for Canyon Lake - Monthly Depth-Integrated Results (Aug – Dec 2018)

Notes:

a - Total Nitrogen = TKN+NO2+NO3

b - Values are site specific dependent upon pH and temperature recorded at each location. Calculated using equation by Thursby (1986).

c - Unionized ammonia concentrations calculated based on ½ detection limit when ammonia nitrogen was ND

NS - Not sampled; DI = Depth integrated; SG = Surface grab; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; m – meters; RL – reporting limit; J- Reported value was detected above the Method Detection Limit (MDL), but below the Reporting Limit (RL)

Table 3-16. Analytical Chemistry Results for Canyon Lake- Monthly Depth-Integrated Results (Feb – June 2019)

Notes:

a - Total Nitrogen = TKN+NO2+NO3

b - Values are site specific dependent upon pH and temperature recorded at each location.  Calculated using equation by Thursby (1986).

c - Unionized ammonia concentrations calculated based on ½ detection limit when ammonia nitrogen was ND

NS - Not sampled; DI = Depth integrated; SG = Surface grab; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; m – meters; RL – reporting limit; J- Reported value was detected above the Method Detection Limit (MDL), but below the Reporting Limit (RL)

Table 3-17. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Each Site in the Main Basin

Notes:

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Concentrations calculated based on ½ detection limit (MDL) when a value in calculation was ND

d - Values calculated using water column mean ammonia, temperature, salinity and pH. Calculated using equation by Thursby (1986).

e - Main Basin values are an average of minimum and maximum values for CL07 and CL08 and an overall mean of all values from both sites.

f – the range of TMDL target thresholds apply to individual samples, not applicable to annual means.  See quarterly reports.

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

NA – Not applicable/ available

NS - Not sampled

DI = Depth integrated; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; m – meters; RL – reporting limit

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

Table 3-18. Analytical Chemistry Results for Canyon Lake- Annual Mean Statistics for Each Site in the East Basin

Notes:

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Concentrations calculated based on ½ detection limit (MDL) when a value in calculation was ND

d - Values calculated using water column mean ammonia, temperature, salinity and pH. Calculated using equation by Thursby (1986).

e - East Basin values are an average of minimum and maximum values for CL09 and CL10 and an overall mean of all values from both sites.

f – the range of TMDL target thresholds apply to individual samples, not applicable to annual means.  See quarterly reports.

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

NA – Not applicable/ available

NS - Not sampled

DI = Depth integrated; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; m – meters; RL – reporting limit

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

Table 3-19. Analytical Chemistry Results for Canyon Lake- Annual Mean Statistics for Both Main and East Basins

Notes:

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Concentrations calculated based on ½ detection limit (MDL) when a value in calculation was ND

d - Values calculated using water column mean ammonia, temperature, salinity and pH. Calculated using equation by Thursby (1986).

e - East Basin values are an average of minimum and maximum values for CL09 and CL10 and an overall mean of all values from both sites.

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

NA – Not applicable/ available

NS - Not sampled

DI = Depth integrated; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; m – meters; RL – reporting limit

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

Figure 3-15. Canyon Lake Analytical Chemistry – Depth-Integrated Means

(Main Basin values represent the mean of Sites CL07 & CL08 , East Basin values represent the mean of Sites CL09 & CL10)

Figure 3-16. Canyon Lake Analytical Chemistry – Depth-Integrated Chlorophyll-a

(Main Basin values represent the mean of Sites CL07 & CL08, East Basin values represent the mean of Sites CL09 & CL10)

Figure 3-17. Canyon Lake Analytical Chemistry- Rolling Averages

Values are calculated by averaging the value from each event with the previous five events (i.e. one year of data) to obtain a rolling average.

## Satellite Imagery

Beginning with the 2015-2016 FY, the TMDL Task Force contracted with satellite vendor EOMAP to conduct remote sensing using LandSat and Sentinel-2 satellite imagery to estimate chlorophyll-a and turbidity concentrations in Lake Elsinore and Canyon Lake.  Using 30-m pixel resolution, this effort produced maps of the lake showing graphical, color-coded images of chlorophyll-a and turbidity concentrations at up to approximately 1,000 unique data points across Canyon Lake and approximately 11,000 unique data points across Lake Elsinore.  This tool provides a snapshot of conditions throughout the entire lake at a given point in time, as opposed to the single data points provided at water quality collection locations and dates; however, the satellite imagery only represents approximately the upper 4 feet of the water column depending on water clarity, and therefore cannot completely replace manual sampling where depth-integrated values are required.

As part of the TMDL compliance monitoring, satellite imagery depicting lake-wide chlorophyll-a and turbidity concentrations in Lake Elsinore and Canyon Lake were generated for each in-lake monitoring event.  Satellite images for each lake during the eight monitoring events evaluated in the report are presented in Figures 3-18 through 3-21. Significant spatial variability in chlorophyll-a is evident, providing a more complete assessment of algal density conditions across each lake.

To quantify the data presented in the satellite images cumulative frequency distribution plots showing lake-wide chlorophyll-a concentrations based on individual pixels from the satellite measurements are provided in Figures 3-22 and 3-23.  Satellite derived median values along with measured chlorophyll-a concentrations collected in-lake are provided for each date showing these single data points relative to concentrations throughout the entire lake.  Median values were derived from satellite imagery data treating each pixel as a unique individual data point.

The satellite images for Lake Elsinore show a generalized lake wide chlorophyll-a concentration increase through December 2018, followed by subsequent decreases February through June 2019.

Chlorophyll-a concentrations in Canyon Lake derived from satellite imagery remained relatively consistent in the main body of the lake.  The eastern arm of the lake did experience some periods of elevated chlorophyll-a values, generally in the far eastern end. Some of this may be due to edge effects as a result of land and water pixels mixing near the edges of the narrow channel. This effect is somewhat diminished during non-summer months when Sentinel-2 satellite data is used, which generates a smaller pixel size (10m resolution), than the LandSat satellite (30m resolution) used during summer months (June – Sept).  However, elevated chlorophyll-a values were still observed in the eastern arm of the lake in October 2018, December 2018, and April 2019.  The large increase in satellite derived chlorophyll-a in February 2019 appears to be a confounding artifact of the very high turbidity in the lake as a result of an extreme rainfall event (approx. 5.5 inches on February 14) occurring five days prior to the sampling event.  This high lake wide turbidity can be seen in the satellite image (Figure 3-21), and in the low Secchi depth measurements relative to the April collection event, as previously discussed.

It was evident that some of the chlorophyll-a concentrations generated from the satellite images for both lakes did not consistently match well with analytical in-lake water samples collected.  In Lake Elsinore this was likely due to the high chlorophyll-a concentrations observed in this water body.  During the initial satellite validation performed over the 2015-2016 sampling season, it was determined that satellite chlorophyll-a estimates tended to diverge from analytical in-lake concentrations when in-lake concentrations were above approximately 100 µg/L (i.e. the satellite estimates remained low as in-lake concentrations increased).  One exception to this general trend was September 11, 2018 where the satellite image shows a slightly greater chlorophyll-a concentration relative to that measured in the samples collected from the lake on the same date.  Chlorophyll-a concentration estimates in Canyon Lake based on the cumulative distribution plots were generally greater than those measured analytically with the exception of December where both measures were relatively close. The distinction based on map colors however appeared show a closer correspondence between satellite and analytically-derived data.  Discussions and sharing of this data with EOMaps will continue to enhance model predictions and assess whether there may be other confounding factors including turbidity not accurately accounted for in this lake. Consistent with the analytical results, however, chlorophyll-a concentration estimates in Canyon Lake using both satellite imagery and analytical measurements were lower than those observed in Lake Elsinore.  The satellite images are also able to provide sense of the relative variability in algae concentrations across the lake which can be rather dramatic and is missed by measuring values from only a few discrete locations.

September 11, 2018

August 24, 2018

July 23, 2018

October 16, 2018

February 19, 2019

December 20, 2018

Figure 3-18.  Satellite Imagery of Chlorophyll-a Concentrations in Lake Elsinore

(Data gaps in July, September, December and February are due to surface cyanobacterial slicks.)

April 9, 2019

June 24, 2019

Figure 3-18 (cont).  Satellite Imagery of Chlorophyll-a Concentrations in Lake Elsinore

(Data gaps in June are due to surface reflection.)

September 11, 2018

July 23, 2018

August 24, 2018

December 20, 2018

February 19, 2019

October 16, 2018

Figure 3-19.  Satellite Imagery of Turbidity Concentrations in Lake Elsinore

(Data gaps in July, September, December, and February are due to surface cyanobacterial slicks.)

June 24, 2019

April 9, 2019

Figure 3-19 (cont).  Satellite Imagery of Turbidity Concentrations in Lake Elsinore

(Data gaps in June are due to surface reflection.)

February 19, 2019

July 23, 2018

September 11, 2018

October 16, 2018

December 20, 2018

August 24, 2018

Figure 3-20.  Satellite Imagery of Chlorophyll-a Concentrations in Canyon Lake

(High cirrus cloud interference caused data gaps in July 2018, and decreased clarity in June 2019.)

June 24, 2019

April 9, 2019

Figure 3-20 (cont).  Satellite Imagery of Chlorophyll-a Concentrations in Canyon Lake

(High cirrus cloud interference caused data gaps in July 2018, and decreased clarity in June 2019.)

July 23, 2018

February 19, 2019

December 20, 2018

October 16, 2018

August 24, 2018

September 11, 2018

Figure 3-21.  Satellite Imagery of Turbidity Measurements Canyon Lake

(High cirrus cloud interference caused data gaps in July 2018, and decreased clarity in June 2019.)

April 9, 2019

June 24, 2019

Figure 3-21 (cont).  Satellite Imagery of Turbidity Measurements Canyon Lake

(High cirrus cloud interference caused data gaps in July 2018, and decreased clarity in June 2019.)

Figure 3-22.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Lake Elsinore Relative to Field Collected Samples

Figure 3-23.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Canyon Lake Relative to Field Collected Samples

Figure 3-23. (cont).  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Canyon Lake Relative to Field Collected Sample

# References

Amec Foster Wheeler. 2016. Quality Assurance Project Plan for Lake Elsinore, Canyon Lake, and San Jacinto River Watershed TMDL Monitoring Program. May 2015.

Amec Foster Wheeler. 2017. Lake Elsinore and Canyon Lake Watersheds Nutrient TMDL Monitoring, 2016-2017 Annual Report.  Prepared for the Lake Elsinore & San Jacinto Watersheds Project Authority, August 2017.

Haley & Aldrich, Inc. 2015. Lake Elsinore and Canyon Lake Nutrient TMDL Compliance Monitoring Work Plan for Lake Elsinore and San Jacinto Watershed Authority. Riverside, CA. April 2016.

Lake Elsinore and San Jacinto Watersheds Authority (LESJWA). 2006. Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan. CA Regional Water Quality Control Board, Santa Ana Region. February.

Montgomery Watson and Elsinore Valley Municipal Water District (MWH/ EVMWD). 2009.  Canyon Lake Bacteria Characterization Study. Technical Memorandum prepared for the Lake Elsinore and Canyon Lake TMDL Task Force.  December 1, 2009

Riverside County Flood Control and Water Conservation District (RCFCWCD). 2019. Rain Gauge Map. http://rcflood.org/RainFallMap.aspx.

Santa Ana Regional Water Quality Control Board. 2007. Resolution Amending the Water Quality Control Plan for the Santa Ana River Basin to Incorporate Nutrient Total Maximum Daily Loads (TMDLs) for Lake Elsinore and Canyon Lake, No. R8-2004-0037.

Santa Ana Regional Water Quality Control Board. 2007. Resolution Approving Plans and Schedules Submitted by the Canyon Lake/Lake Elsinore TMDL Task Force and Individual Discharger Groups Pursuant to the Lake Elsinore and Canyon Lake Nutrient Total Maximum Daily Loads Specified in the Water Quality Control Plan for the Santa Ana River Basin. Resolution R8-2007-0083.

Thursby, C.B. 1986. Memorandum to David J. Hansen, U.S. EPA, Narragansett, Rhode Island.

This page intentionally left blank.

Based on federally listed Riverside fairy shrimp (Streptocephalus woottoni) in adjacent wetlands.

The 303d listing for bacteria in Canyon Lake was addressed by the TMDL Task Force and Elsinore Valley Municipal Water District (EVMWD) through an enhanced special bacteria indicator monitoring study in 2009 by Montgomery Watson (EVMWD/ MWH, 2009).  The results of this study found no exceedances relative to recreational contact water quality standards.

The potential impact of this Holy Fire sediment plume is being evaluated under a separate study funded by the State Water Quality Control Board Cleanup and Abatement Account through the City of Lake Elsinore.

The Sentinel-2 satellite cannot be used during summer months due to a glare from the sun caused by the angle of satellite viewing, and thereby reducing the image quality.