Lake Elsinore and Canyon Lake

Watersheds Nutrient TMDL Monitoring

2020-2021 Annual Report – DRAFT

Prepared for:

Lake Elsinore & San Jacinto Watersheds Project Authority

11615 Sterling Avenue

Riverside, California 92503

Prepared by:

Wood Environment and Infrastructure Solutions Inc.

9177 Sky Park Court

San Diego, CA 92123

August 15, 2021

TABLE OF CONTENTS

Page

1.0	Introduction	1

1.1	Background	1

2.0	San Jacinto River Watershed-Wide Monitoring	3

2.1	Summary of 2020-2021 Wet Weather Watershed Monitoring and Nutrient Loads	4

2.2	Historical Wet Weather Watershed Monitoring and Nutrient Loads	4

2.3	Monitoring Strategy	7

2.4	Monitoring Stations and Stream Gauge Locations	7

2.5	Stream Gauge Records	9

2.6	Sampling Strategy	12

2.7	San Jacinto River Watershed Monitoring Events	13

2.8	San Jacinto River Watershed Annual Water Quality Summary	14

2.8.1	Summary of Monitoring Data – Salt Creek at Murrieta Road	14

2.8.2	Summary of Monitoring Data – San Jacinto River at Goetz Road	18

2.8.3	Summary of Monitoring Data – San Jacinto River at Ramona Expressway	22

2.8.4	Summary of Monitoring Data – Canyon Lake Spillway	22

2.9	San Jacinto River Watershed Rainfall Records	27

3.0	In-Lake Monitoring	28

3.1	Background	28

3.2	Lake Elsinore Monitoring	28

3.2.1	Sampling Station Locations and Frequency	28

3.2.2	Sampling Methods	30

3.2.3	Water Quality Summary	31

3.3	Canyon Lake Monitoring	47

3.3.1	Sampling Station Locations and Frequency	47

3.3.2	Sampling Methods	49

3.3.3	Water Quality Summary	50

3.4	Satellite Imagery	69

4.0	References	83

LIST OF APPENDICES

Appendix A Watershed Analytical Reports

Appendix B Water quality profile tables

Appendix C Lake Monitoring Analytical Reports

Appendix D Satellite Data Reports

Appendix E Current Data in Historical Context

LIST OF TABLES

Table 2-1. Summary of 2020-2021 Monitoring	4

Table 2-2. Summary of Historical Annual Mean Storm Concentrations	5

Table 2-3. Summary of Historical Estimated Annual Loads	6

Table 2-4. San Jacinto River Watershed Monitoring Stations	7

Table 2-5. Summary of Stream Gauge Data (July 2020 through June 2021)	9

Table 2-6. Water Quality Concentrations at Salt Creek at Murrieta Road	16

Table 2-7. Water Quality Event and Annual Loads at Salt Creek at Murrieta Road	17

Table 2-8. Water Quality Concentrations at San Jacinto River at Goetz Road	20

Table 2-9. Water Quality Event and Annual Loads at San Jacinto River at Goetz Road	21

Table 2-10. Water Quality Concentrations at Canyon Lake Spillway	24

Table 2-11. Water Quality Event and Annual Loads at Canyon Lake Spillway	25

Table 2-12. San Jacinto River Watershed Rainfall Gauges	27

Table 2-13. Summary Rainfall Data (July 2020 to June 2021)	27

Table 3-1.  Lake Elsinore TMDL Monitoring Locations	30

Table 3-2.  2020-2021 In-lake Analytical Constituents and Methods for Lake Elsinore	30

Table 3-3. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2020 Monthly Means for Each Site (July – Dec 2020)	32

Table 3-4. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2021 Monthly Means for Each Site (February – June 2021)	32

Table 3-5. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2020-2021 Annual Mean Statistics for Each Site	34

Table 3-6.  Monthly Analytical Chemistry Results for Lake Elsinore in 2020-2021	42

Table 3-7. Analytical Chemistry Summary for Lake Elsinore – Annual Mean Statistics (2020-2021)	43

Table 3-8.  Canyon Lake TMDL Monitoring Locations	47

Table 3-9.   In-lake Analytical Constituents and Methods for Canyon Lake (2020-2021)	50

Table 3-10. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (August – December 2020)	52

Table 3-11. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (February – June 2021)	53

Table 3-12. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (August – December 2020)	54

Table 3-13. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (February – June 2021)	54

Table 3-14. In-Situ Water Quality Parameter Measurements for Canyon Lake - Annual Mean Statistics for Each Site (August 2020 – June 2021) and Comparison to Previous Monitoring Events	55

Table 3-15. Analytical Chemistry Results for Canyon Lake - Monthly Depth-Integrated Results (Aug – Dec 2020)	62

Table 3-16. Analytical Chemistry Results for Canyon Lake- Monthly Depth-Integrated Results (Feb – June 2021)	63

Table 3-17. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Each Site in the Main Basin	64

Table 3-18. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Each Site in the East Basin	65

Table 3-19. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Both Main and East Basins	66

LIST OF FIGURES

Figure 2-1. San Jacinto River Watershed Monitoring Stations	8

Figure 2-2. Site 3 – Salt Creek at Murrieta Road – Daily Stream Gauge Records	10

Figure 2-3. Site 4 – San Jacinto River at Goetz Road – Daily Stream Gauge Records	10

Figure 2-4. Site 30 – Canyon Lake Spillway – Daily Stream Gauge Records	11

Figure 2-5. Site 6 – San Jacinto River at Ramona Expressway – Daily Stream Gauge Records	11

Figure 2-6. Site 1 – San Jacinto River at Cranston Guard Station – Daily Stream Gauge Records	12

Figure 2-7. Storm Event at Salt Creek at Murrieta Road (January 29-31, 2021)	15

Figure 2-8. Storm Event at Salt Creek at Murrieta Road (March 10-12, 2021)	15

Figure 2-9. Hydrograph of First Storm Event at Salt Creek at Murrieta Road (January 29 -31, 2021)	17

Figure 2-10. Hydrograph of Second Storm Event at Salt Creek at Murrieta Road (March 10 - 12, 2021)	18

Figure 2-11. Storm Event at San Jacinto River at Goetz Road (January 29 - 31, 2021)	19

Figure 2-12. Storm Event at San Jacinto River at Goetz Road (March 10 - 12, 2021)	19

Figure 2-13. Hydrograph of First Storm Event at San Jacinto River at Goetz Road  (January 29-31, 2021)	21

Figure 2-14. Hydrograph of Second Storm Event at San Jacinto River at Goetz Road  (March 10 - 12, 2021)	22

Figure 2-15. Storm Event Sampling Below the Canyon Lake Spillway (March 10 - 15, 2021)	23

Figure 2-16. Canyon Lake Spillway (March 10 - 15, 2021)	23

Figure 2-17. Hydrograph of First Storm Event at Canyon Lake Spillway (January 29-February 1, 2021)	25

26

Figure 2-18. Hydrograph of Second Storm Event at Canyon Lake Spillway (March 10-15, 2021)	26

Figure 2-19. Canyon Lake Level at Railroad Canyon Dam Spillway	26

Figure 3-1. Lake Elsinore Sampling Locations	29

Figure 3-2. Water Column Mean Dissolved Oxygen (DO) Rolling Average – Lake Elsinore for Site LE02	35

Figure 3-3. In-Situ Physical Water Quality Parameters - Lake Elsinore - Site LE01	36

Figure 3-4. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE02	37

Figure 3-5. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE03	38

Figure 3-6. Monthly Lake-wide Mean of In-Situ Physical Water Quality Parameters – Mean of All Three Stations	39

Figure 3-7. In- Situ Water Clarity Using a Secchi Disk - Lake Elsinore Site LE02	40

Figure 3-8. Lake Elsinore Analytical Chemistry – Depth-Integrated Means at Site LE02 (July 2020-June 2021)	44

Figure 3-9. Lake Elsinore Analytical Chemistry – Total Nitrogen and Phosphorus Rolling Averages (July 2020 – June 2021)	45

Figure 3-10. Lake Elsinore Analytical Chemistry – Depth-Integrated and Surface Chlorophyll-a at Site LE02	46

Figure 3-11. Canyon Lake Sampling Locations	48

Figure 3-12. Mean In-Situ Physical Water Quality Parameters – Canyon Lake Main Basin	56

Figure 3-13. Mean In-Situ Physical Water Quality Parameters - Canyon Lake East Basin	57

Figure 3-14. Rolling Average Concentrations of Dissolved Oxygen in the Epilimnion and Hypolimnion of Canyon Lake	58

Figure 3-15. Rolling Average Concentration of Dissolved Oxygen Across the Full Vertical Water Column in Canyon Lake	58

Figure 3-16. In-Situ Water Clarity Using a Secchi Disk– Main and East Basins	59

Figure 3-17. Canyon Lake Analytical Chemistry – Depth-Integrated Means	67

Figure 3-18. Canyon Lake Analytical Chemistry – Depth-Integrated Chlorophyll-a	68

Figure 3-19. Canyon Lake Analytical Chemistry- Rolling Averages	69

Figure 3-20.  Satellite Imagery of Chlorophyll-a Concentrations in Lake Elsinore	72

Figure 3-21.  Satellite Imagery of Turbidity Concentrations in Lake Elsinore	74

Figure 3-22.  Satellite Imagery of Chlorophyll-a Concentrations in Canyon Lake	76

Figure 3-23.  Satellite Imagery of Turbidity Measurements Canyon Lake	78

Figure 3-24.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Lake Elsinore Relative to Measured Chlorophyll-a in Field Collected Samples	80

Figure 3-25.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Canyon Lake Relative to Measured Chlorophyll-a in Field Collected Samples	81

Acronyms and Abbreviations

| µg/L | micrograms per liter |
| --- | --- |
| µS/cm | microSiemens per centimeter |
| Basin Plan | Water Quality Control Plan for the Santa Ana River Basin |
| CCC | criterion continuous concentration |
| cf | cubic feet |
| cfs | cubic feet per second |
| CMC | criterion maximum concentration |
| DI | Depth-integrated |
| DO | dissolved oxygen |
| EMC | event mean concentration |
| Epi | epilimnion |
| EVMWD | Elsinore Valley Municipal Water District |
| Forest Service | San Bernardino Nation Forest Service |
| FY | fiscal year |
| Hypo | hypolimnion |
| J | concentration between MDL and RL |
| kg | kilogram |
| LA | load allocation |
| LESJWA | Lake Elsinore and San Jacinto Watersheds Authority |
| MDL | Method detection limit |
| Mgal | million gallons of water |
| mg/L | milligrams per liter |
| NA | not applicable |
| ND | non-detect |
| NM:LE | not measured lab error |
| NPDES | National Pollutant Discharge Elimination System |
| NS | not sampled |
| NWS | National Weather Service |
| QAPP | Quality Assurance Project Plan |
| RCFC&WCD | Riverside County Flood Control and Water Conservation District |
| RL | Reporting limit |
| RWQCB | Regional Water Quality Control Board, Santa Ana Region |
| SAWPA | Santa Ana Watershed Project Authority |
| SM | Standard Method |
| Surf | Surface sample (0-2 meter composite) |
| TDS | Total Dissolved Solids |
| TMDL | Total Maximum Daily Load |
| TMDL Task Force | Lake Elsinore and Canyon Lake TMDL Task Force |
| US EPA | U.S. Environmental Protection Agency |
| USGS | United States Geological Survey |
| WLA | waste load allocation |

This page intentionally left blank.

# Introduction

The following document summarizes results of compliance monitoring required in support of the Lake Elsinore and Canyon Lake Nutrient Total Maximum Daily Load (TMDL) for the 2020-2021 fiscal year (FY). The monitoring was performed according to the Lake Elsinore & Canyon Lake Nutrient TMDL Monitoring Quality Assurance Project Plan (QAPP) (Amec Foster Wheeler, September 2016), and the associated Compliance Monitoring Work Plan (Haley & Aldrich, Inc., July 2016).

## Background

Lake Elsinore is a natural freshwater lake in southern California that provides a variety of natural habitats for terrestrial and aquatic species.  The beneficial uses of the lake include water contact recreation (REC1), non-water contact recreation (REC2), commercial and sportfishing (COMM), warm freshwater habitat (WARM), wildlife habitat (WILD), and rare, threatened or endangered species (RARE).  While being a natural lake, the lake has been modified in various ways to enhance its recreational use and aquatic habitat, including creation of a levee at the lake’s south end to increase the water depth / reduce evaporation, and water in the lake is supplemented with approximately 6 million gallons per day of recycled water from Elsinore Valley Municipal Water District (EVMWD).  Canyon Lake was constructed in 1928 as the Railroad Canyon Reservoir.  It is located approximately two miles upstream of Lake Elsinore and water spilled from Canyon Lake is a main source of water for Lake Elsinore during wet years.  The beneficial uses of Canyon Lake include municipal and domestic water supply (MUN), agricultural supply (AGR), groundwater recharge (GWR), body contact recreation (REC1), non-body contact recreation (REC2), commercial and sportfishing (COMM), warm freshwater aquatic habitat (WARM), and wildlife habitat (WILD).  The beneficial uses of COMM and RARE in Lake Elsinore and COMM in Canyon Lake were approved by the California Regional Water Quality Control Board, Santa Ana Region (RWQCB) as an amendment to the Water Quality Control Plan for the Santa Ana River Basin (Basin Plan) under resolution R8-2017-0019 on June 16, 2017, and became effective on October 15, 2018 after being approved by US EPA.

In 1994, Lake Elsinore and Canyon Lake were first listed by the RWQCB on its Clean Water Act Section 303(d) list of impaired waterbodies.  Both lakes remain on the latest approved 303(d) list, Res. No. 2017-0059. Current impairments identified for these waters included excessive levels of nutrients in both lakes, as well as organic enrichment/low dissolved oxygen (DO), sedimentation/siltation, unknown causes of toxicity, and PCBs/DDTs in Lake Elsinore.  The Clean Water Act Section 303(d) requires the development and implementation of a TMDL for waters that do not or are not expected to meet water quality standards (beneficial uses, water quality objectives). In 2000, the RWQCB initiated the development of TMDLs for nutrients for Lake Elsinore and Canyon Lake.

In December 2004, the RWQCB adopted amendments to the Basin Plan to incorporate TMDLs for nutrients in Canyon Lake and Lake Elsinore.  The amendments were subsequently approved by US EPA on September 30, 2005.  The Basin Plan Amendment specifies, among other things, monitoring recommendations to measure progress towards attainment of TMDL thresholds and associated waste load allocations (WLAs) and monitoring to measure compliance towards in-lake numeric water quality targets. Numeric targets have been established and incorporated in the TMDL for nutrients (total nitrogen, phosphorous, and ammonia), DO, and chlorophyll-a; however, the ultimate compliance goal for beneficial uses in both lakes is to reduce eutrophication, which can negatively affect biological communities, result in fish kills, and impact recreational use. The recommendations outlined in RWQCB Resolution No. R8-2004-0037 required stakeholders to develop management plans and conduct long-term monitoring and implementation programs aimed at reducing nutrient loads to Lake Elsinore and Canyon Lake.  Task 4 of the adopted Lake Elsinore and Canyon Lake TMDL Amendment required stakeholders to prepare and implement a Nutrient Monitoring Program. The program was to include the following:

- A watershed-wide monitoring program to determine compliance with interim and/or final nitrogen and phosphorus allocations; compliance with the nitrogen and phosphorus TMDL, and load allocations (LAs), including WLAs.

- A Lake Elsinore in-lake nutrient monitoring program to determine compliance with interim and final nitrogen, phosphorus, chlorophyll-a, and DO numeric targets.

- A Canyon Lake in-lake nutrient monitoring program to determine compliance with interim and final nitrogen, phosphorus, chlorophyll-a, and DO numeric targets.

- An annual report summarizing the data collected for the year and evaluating compliance with the TMDL, due August 15 of each year.

Since August 2001, the Lake Elsinore and San Jacinto Watershed Authority (LESJWA) has been working with local stakeholders and the RWQCB to identify the source of nutrients impairing each lake and evaluate the impacts to water quality and beneficial uses incurred from nutrient sources.

At that time, LESJWA contracted with the State to serve as a neutral facilitator for the RWQCB to assist in formation of a TMDL workgroup and assist the workgroup in participating with the RWQCB in the development and definition of the TMDLs.

After adoption of the Lake Elsinore and Canyon Lake nutrient TMDLs on December 20, 2004, stakeholders named in the TMDLs began the process to create a formal cost sharing body, or Task Force, to implement a number of tasks included in the TMDLs.

In November 2006, stakeholders finalized an agreement to form the Lake Elsinore and Canyon Lake TMDL Task Force (hereafter “TMDL Task Force”).  The TMDL Task Force consists of representatives from local cities, Riverside County, agriculture and dairy, and the regulatory community.  At the request of the stakeholders and RWQCB, LESJWA (staffed by the Santa Ana Watershed Project Authority or “SAWPA”) serves as administrator of the TMDL Task Force and oversees the TMDL implementation for Lake Elsinore and Canyon Lake.

LESJWA, in support of the TMDL Task Force, provided funding to meet the requirement of the TMDL by developing a single comprehensive watershed-wide nutrient Monitoring Plan. The Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan was approved by the RWQCB in March 2006, and subsequently implemented by the TMDL Task Force starting in April 2006 through October 2012.  During this time frame, in-lake monitoring for both lakes was conducted through the EVMWD National Pollutant Discharge Elimination System (NPDES) compliance program (Order No. R8-2005-0003, NPDES No. CA8000027, Regional Water Reclamation Plant, Lake Elsinore, Riverside County).  On October 26, 2012, the RWQCB adopted a resolution (Resolution No. R8-2012-0052) granting the TMDL Task Force a temporary suspension of in-lake TMDL monitoring programs to achieve cost savings that were then applied to implementing lake improvement projects aimed at reducing nutrient impacts in Canyon Lake and Lake Elsinore. As a result, the Lake Elsinore and Canyon Lake Nutrient TMDL field compliance monitoring was not conducted during the 2013-2014 and 2014-2015 fiscal year (FY) cycles.

The in-lake and watershed-wide water quality monitoring for both lakes was resumed in July 2015 as Phase II of the Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Program moving forward.  A revised Monitoring Work Plan (Haley & Aldrich 2016) and companion Quality Assurance Project Plan (Amec Foster Wheeler 2016) were prepared and approved by the RWQCB in October 2016. The primary objectives considered in developing the Nutrient TMDL Compliance Monitoring Work Plan were to: 1) evaluate the status and trends toward achieving response targets in both lakes through in-lake monitoring; and 2) to distinguish and quantify the external pollutant loading originating from the watershed above the lakes through stormwater monitoring of the major upstream inputs to Canyon Lake.  Additional objectives of the monitoring are to support the stormwater compliance activities underway by other entities in the watershed, including the reissuance of the Riverside County Municipal Stormwater National Pollutant Discharge Elimination Systems Permit (Order R8-2010-0033, Municipal Separate Storm Sewer System (MS4) Permit), and land use monitoring requirements related to the Conditional Waiver for Agricultural Discharges (Adopted Order R8-2017-0023; Amending Order R8-2016-0003).  The results of the 2020-2021 FY in-lake and watershed monitoring efforts are summarized herein.

# San Jacinto River Watershed-Wide Monitoring

The primary objectives of the Phase II San Jacinto River Watershed Monitoring Program are as follows:

- Determine the total nutrient loads into Lake Elsinore and Canyon Lake from their tributaries (i.e., San Jacinto River and Salt Creek).

- Determine the total nutrient load from various sources categorized by land use types, namely, agricultural, urban runoff, and open space sources which drain into the above-mentioned tributaries.

- Provide water quality data for watershed model updates.

Watershed monitoring and reporting was performed by Alta Environmental DBA NV5 of San Diego, California.

## Summary of 2020-2021 Wet Weather Watershed Monitoring and Nutrient Loads

A summary of the measured concentrations and estimated annual nutrient loads derived from each of the three monitored locations for the period of July 1, 2020 through June 30, 2021, is presented in Table 2-1. A more detailed account, including storm hydrographs and event loads are presented in the following sections for each monitoring location.

Table 2-1. Summary of 2020-2021 Monitoring

| Number and Location Description | Total Annual Flow a (Mgal) | Annual Event Mean Storm Concentration (mg/L) |  | Estimated Annual Load (kg) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Total Nitrogen | Total Phosphorus | Total Nitrogen | Total Phosphorus |
| Site 3 - Salt Creek at Murrieta Road (USGS 11070465) | 255 | 1.9 | 0.39 | 1,902 | 396 |
| Site 4 - San Jacinto River at Goetz Road (USGS 11070365) | 519 | 1.9 | 0.48 | 3,794 | 992 |
| Site 6 - San Jacinto River at Ramona Expressway b (USGS 11070210) | 0 | Not Measured b | Not Measured b | Not Measured b | Not Measured b |
| Site 30 - Canyon Lake Spillway c (USGS 11070500) | 878 | 1.7 | 0.05 | 5,626 | 175 |

a - Flow data after 04/27/2021 are provisional and may be subject to change.

b - No flows originating from the upper watershed were observed at the TMDL monitoring location just downstream of Mystic Lake and no sampling was conducted.

c –The USGS stream gauge at Site 30 (USGS 11070500) is located downstream of Canyon Lake on the San Jacinto River close to the river entrance to Lake Elsinore.  This downstream location is influenced by local urban runoff and groundwater seepage in addition to the flows from Canyon Lake. In addition, runoff from other local tributaries into Lake Elsinore are not included in this table.

Mgal = million gallons; 1 million gallons = 133,680 cubic feet = 3,785,412 L; mg/L = milligrams per liter; kg = kilograms; USGS = United States Geological Survey.

## Historical Wet Weather Watershed Monitoring and Nutrient Loads

A summary of the historical water quality monitoring data for the period of July 1, 2011 through June 30, 2021, is presented in Table 2-2 and Table 2-3 for total nitrogen and total phosphorus. In general, the monitoring locations only flow during storm events and the storm flows account for the estimated annual load of nutrients.

Table 2-2. Summary of Historical Annual Mean Storm Concentrations

| Monitoring Year | Site 3 - Salt Creek at Murrieta Road |  | Site 4 - San Jacinto River at Goetz Road |  | Site 30 - Canyon Lake Spillway |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total Nitrogen (mg/L) | Total Phosphorus (mg/L) | Total Nitrogen (mg/L) | Total Phosphorus (mg/L) | Total Nitrogen (mg/L) | Total Phosphorus (mg/L) |
| 2011-2012 | 1.9 | 0.3 | 2.2 | 0.5 | NS | NS |
| 2012-2013 | 1.9 | 0.3 | 2.1 | 0.5 | NS | NS |
| 2013-2014 | 2.7 | 0.9 | 1.8 | 0.6 | NS | NS |
| 2014-2015 | 2.2 | 0.5 | 1.8 | 0.4 | NS | NS |
| 2015-2016 | 2.5 | 0.5 | 2.4 | 1.4 | NS | NS |
| 2016-2017 | 2.1 | 0.6 | 2.0 | 1.2 | 1.9 | 0.4 |
| 2017-2018 | 2.7 | 0.4 | 2.0 | 0.4 | NS | NS |
| 2018-2019 | 2.4 | 0.4 | 1.7 | 0.6 | 1.4 | 0.2 |
| 2019-2020 | 2.4 | 0.6 | 1.8 | 0.7 | 1.1 | 0.16 |
| 2020-2021 | 1.9 | 0.4 | 1.9 | 0.5 | 1.7 | 0.05 |

NS – Not sampled when Canyon Lake does not overtop the Canyon Lake Spillway. The USGS stream gauge at Site 30 (USGS 11070500) is located downstream of Canyon Lake on the San Jacinto River close to the river entrance to Lake Elsinore.  This downstream location is influenced by local urban runoff and groundwater seepage in addition to the flows from Canyon Lake. In addition, runoff from other local tributaries into Lake Elsinore are not included in this table.

Table 2-3. Summary of Historical Estimated Annual Loads

| Monitoring Year | Site 3 - Salt Creek at Murrieta Road |  |  | Site 4 - San Jacinto River at Goetz Road |  |  | Site 30 - Canyon Lake Spillway |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Annual Flow (Mgal) | Total Nitrogen (kg) | Total Phosphorus (kg) | Total Annual Flow (Mgal) | Total Nitrogen (kg) | Total Phosphorus (kg) | Total Annual Flow (Mgal) | Total Nitrogen (kg) | Total Phosphorus (kg) |
| 2011-2012a | 743 | 5,371 | 1,099 | 881 | 6,370 | 3,535 | 1,290 | 5,474 | 3,062 |
| 2012-2013 | 147 | 1,025 | 180 | 424 | 3,341 | 822 | 114 | NS | NS |
| 2013-2014 | 411 | 4,268 | 1,409 | 484 | 3,252 | 1,178 | 148 | NS | NS |
| 2014-2015 | 511 | 4,661 | 1,257 | 570 | 3,932 | 1,041 | 196 | NS | NS |
| 2015-2016 | 515 | 5,647 | 1,447 | 872 | 7,926 | 4,624 | 476 | NS | NS |
| 2016-2017 | 1,596 | 12,366 | 4,026 | 2,802 | 21,651 | 14,403 | 4,850 | 33,759 | 6,637 |
| 2017-2018 | 271 | 2,586 | 482 | 393 | 3,055 | 810 | 117 | NS | NS |
| 2018-2019 | 1,394 | 12,213 | 2,266 | 3,208 | 20,457 | 7,409 | 5,893 | 32,832 | 5,416 |
| 2019-2020 | 1,645 | 14,792 | 3,705 | 3,290 | 23,337 | 8,660 | 4,497 | 18,762 | 2,635 |
| 2020-2021 | 255 | 1,902 | 396 | 519 | 3,794 | 992 | 878 | 5,626 | 175 |

NS – Not sampled when Canyon Lake does not overtop the Canyon Lake Spillway. The USGS stream gauge at Site 30 (USGS 11070500) is located downstream of Canyon Lake on the San Jacinto River close to the river entrance to Lake Elsinore.  This downstream location is influenced by local urban runoff and groundwater seepage in addition to the flows from Canyon Lake. In addition, runoff from other local tributaries into Lake Elsinore are not included in this table.

a - Sum of January 1, 2011 to June 30, 2012. All other monitoring year dates are July 1 to June 30.

## Monitoring Strategy

Phase II of the San Jacinto River Watershed Monitoring Program follows the guidelines detailed in the Lake Elsinore and Canyon Lake Nutrient TMDL Compliance Monitoring Plan.  The Phase II San Jacinto River Watershed Monitoring Program sampling activities during the 2020-2021 monitoring period included collection of samples during three storm events at the designated monitoring stations throughout the San Jacinto River Watershed.  Average nutrient concentrations during these three events were used to calculate mass loading during remaining wet weather events that were not monitored to derive total estimated annual mass loads throughout the monitoring year.

## Monitoring Stations and Stream Gauge Locations

To monitor TMDL compliance, five sampling stations were carefully selected to reflect various types of land uses within the San Jacinto River Watershed. Sampling of these locations began in 2006.  Sampling station locations were deliberately set up to be within the vicinity of United States Geological Survey (USGS) stream gauge stations.  The sampling stations are listed in Table 2-4 below and shown on Figure 2-1.

Three of the five sites (Station IDs 745, 759, and 741) were selected because they are indicative of inputs to Canyon Lake originating from the main stem of the San Jacinto River, Salt Creek, and the watershed above Mystic Lake. The sampling location along the San Jacinto River at Ramona Expressway (Station 741) is located downgradient of Mystic Lake, an area of land subsidence. Flow has not been observed at this location since a strong El Niño event in the mid-1990s. Because of the active subsidence, this monitoring station is not expected to flow except under extremely high rainfall conditions.

Table 2-4. San Jacinto River Watershed Monitoring Stations

| Station ID | USGS Station ID | Agency | Site Number and Location Description |
| --- | --- | --- | --- |
| 745 | 11070465 | USGS | Site 3 - Salt Creek at Murrieta Road |
| 759 | 11070365 | USGS | Site 4 - San Jacinto River at Goetz Road |
| 741 | 11070210 | USGS | Site 6 - San Jacinto River at Ramona Expressway |
| 841 | 11070500 | USGS | Site 30 - Canyon Lake Spillway |
| 792a | 11069500 | USGS | Site 1 - San Jacinto River at Cranston Guard Station |

a - The Cranston Guard Station (Station 792) was monitored between 2007 and 2011 by the San Bernardino National Forest Service in accordance with their agreement for in-lieu obligations to the Task Force. In 2012, the Forest Service pulled out of the Task Force and no longer provides monitoring support.

Figure 2-1. San Jacinto River Watershed Monitoring Stations

The fourth site, located below the Canyon Lake Dam (Station ID 841), is indicative of loads entering Lake Elsinore from Canyon Lake and the upstream watershed when the water level overtops the Railroad Canyon Dam Spillway. This site only represents a portion of the total load into Lake Elsinore from upstream of Canyon Lake Dam and does not include runoff from the local watershed. The Railroad Canyon Dam Spillway elevation at Canyon Lake is 1,381.76 feet. Samples are collected from this location during storm events that create lake levels that overtop the dam spillway elevation.  The Canyon Lake level is publicly available at the following website:

https://www.evmwd.com/who-we-are/lake-levels

The fifth site at the Cranston Guard Station site on the San Jacinto River (Station 792) was only monitored between 2007 and 2011 by the San Bernardino National Forest Service whom no longer provides monitoring support.

## Stream Gauge Records

The USGS monitor stream flow from several gauging stations in the San Jacinto River Watershed. Stream gauging stations maintained and operated for Phase II of the San Jacinto River Watershed Monitoring Program are shown in Figure 2-1 and identified in Table 2-4.

The data record captured per USGS stream gauge is publicly available at the USGS website, where data for the specific gauge numbers provided in Table 2-5 can be found:

http://waterdata.usgs.gov/ca/nwis/current/?type=flow

A summary of the stream gauge data recorded at each of the stations with measured flow for the monitoring period of July 1, 2020 through June 30, 2021 is presented in Table 2-5 and visually presented in Figure 2-2 through Figure 2-6.  The mean monthly flows reported in Table 2-5 characterize the average instantaneous flow rate at the USGS stations.  In general, the flows are only observed during wet weather storm events and dry weather flows are not observed from each of the USGS stations. The flow data are downloaded from the USGS website and are considered provisional for approximately six months; therefore, flow data presented after April 27, 2021 in this report are provisional.  The provisional data provided by the USGS are subject to change and are not citable until reviewed and approved by the USGS.

Table 2-5. Summary of Stream Gauge Data (July 2020 through June 2021)

| July 2020-June 2021 Mean Monthly Flow (cfs) a | Site 3 - Salt Creek at Murrieta Road (11070465c) | Site 4 - San Jacinto River at Goetz Road (11070365 c) | Site 6 - San Jacinto River at Ramona Expressway b (11070210 c) | Site 30 - Canyon Lake Spillway (11070500 c) | Site 1 - San Jacinto River at Cranston Guard Station (11069500 c) |
| --- | --- | --- | --- | --- | --- |
| July | 0.00 | 0.00 | 0.00 | 0.02 | 0.23 |
| August | 0.00 | 0.00 | 0.00 | 0.00 | 0.07 |
| September | 0.00 | 0.00 | 0.00 | 0.00 | 0.05 |
| October | 0.00 | 0.00 | 0.00 | 0.09 | 0.64 |
| November | 0.00 | 0.00 | 0.00 | 0.63 | 0.31 |
| December | 3.17 | 4.33 | 0.00 | 1.22 | 0.83 |
| January | 6.56 | 16.11 | 0.00 | 14.63 | 1.27 |
| February | 0.03 | 0.20 | 0.00 | 6.54 | 1.85 |
| March | 2.97 | 5.26 | 0.00 | 20.11 | 4.06 |
| April | 0.00 | 0.00 | 0.00 | 0.78 | 0.65 |
| May | 0.00 | 0.00 | 0.00 | 0.26 | 0.17 |
| June | 0.00 | 0.00 | 0.00 | 0.00 | 0.04 |
| Mean Annual Flow (cfs) | 1.08 | 2.21 | 0.00 | 3.70 | 0.84 |

Notes:

a - This value characterizes the average instantaneous flow rate at the USGS station during both dry and wet weather conditions in a given month. Flow data 04/27/2021 are provisional and may be subject to change.

b - No flows originating from the upper watershed were observed at the TMDL monitoring location just downstream of Mystic Lake.

c – USGS gauge number

cfs = cubic feet per second.

Figure 2-2. Site 3 – Salt Creek at Murrieta Road – Daily Stream Gauge Records

Figure 2-3. Site 4 – San Jacinto River at Goetz Road – Daily Stream Gauge Records

Figure 2-4. Site 30 – Canyon Lake Spillway – Daily Stream Gauge Records

Figure 2-5. Site 6 – San Jacinto River at Ramona Expressway – Daily Stream Gauge Records

Figure 2-6. Site 1 – San Jacinto River at Cranston Guard Station – Daily Stream Gauge Records

## Sampling Strategy

Phase II of the San Jacinto River Watershed Monitoring Program includes collecting water quality samples during two storm events at the designated monitoring stations throughout the San Jacinto River Watershed. Throughout the wet weather monitoring period from October 1, 2020 to May 31, 2021, the National Weather Service (NWS) forecasts were monitored to determine when storm events met the mobilization criteria. The mobilization criteria for sampling requires a NWS quantitative precipitation forecast greater than a 1.0-inch forecast within 24 hours from October 1 through December 31, and greater than an 0.5-inch forecast within 24 hours from January 1 through May 31.

Flow-weighted composite samples were collected during two storm events at the designated monitoring stations. Discrete sample aliquots were collected over the rising limb (increasing flow) and the falling limb (decreasing flow) of the hydrograph using automatic sampling equipment (e.g., ISCO autosamplers). The first sample aliquot was taken at or shortly after the time that storm water runoff began, and each subsequent aliquot of equal volume was collected at intervals of approximately 2 hours across the hydrograph, depending on the forecasted size of the storm event.  Flow rates and volumes were based on data from USGS stream gauges located near the sampling stations. Upon completion of sampling, field teams downloaded the USGS flow data and subsampled each discrete sample to create a single flow-weighted composite sample for laboratory analysis.

The following protocols were applied:

- Sampling commenced once flow was established in the channel.

- Field measurements (temperature, pH, conductivity, dissolved oxygen, and turbidity) were recorded in the field during the rising limb of the hydrograph using portable calibrated YSI multi-parameter meters, or equivalent.

- Biochemical Oxygen Demand and Chemical Oxygen Demand were analyzed for the first discrete grab sample only.

Sampling and analysis followed the guidelines detailed in the Lake Elsinore and Canyon Lake Nutrient TMDL Compliance Monitoring Plan (Haley & Aldrich, Inc., July 2016). More detail regarding the sampling approach (e.g., compositing, sample naming conventions) are described in the Lake Elsinore and Canyon Lake Nutrient TMDL Compliance QAPP (Amec Foster Wheeler, September 2016). These documents are available at the following website:

https://sawpa.org/task-forces/lake-elsinore-and-canyon-lake-tmdl-task-force/#monitoring-program

Samples for all analytical chemistry measurements were submitted to Babcock Laboratories Inc. located in Riverside, California and Weck Laboratories Inc. located in Industry, California.

## San Jacinto River Watershed Monitoring Events

Water quality samples were collected during two storm events that met the mobilization criteria during the wet weather monitoring period from October 1, 2020 to May 31, 2021, which included a total of two events for each station, except San Jacinto River at Ramona Expressway.

The first monitoring event occurred on January 29, 2021 through February 1, 2021. Water quality samples were collected at Salt Creek at Murrieta Road (Station ID 745), San Jacinto River at Goetz Road (Station ID 759), and Canyon Lake Spillway (Station ID 841). A peak flow of 158 cubic feet per second (cfs) was recorded at Salt Creek at Murrieta Road (Station ID 745), a peak flow of 568 cfs was recorded at San Jacinto River at Goetz Road (Station ID 759), and a peak flow of 332 cfs was recorded at Canyon Lake Spillway (Station ID 841). No flows were recorded at the San Jacinto River at Ramona Expressway (Station ID 741).  A total of 0.76 to 1.08 inches of rainfall was recorded in the region during this storm (RCFCWCD 2021).

The second monitoring event occurred on March 10, 2021 through March 15, 2021. Water quality samples were collected at Salt Creek at Murrieta Road (Station ID 745), San Jacinto River at Goetz Road (Station ID 759), and Canyon Lake Spillway (Station ID 841).  A peak flow of 71 cfs was recorded at Salt Creek at Murrieta Road (Station ID 745), a peak flow of 53 cfs was recorded at San Jacinto River at Goetz Road (Station ID 759), and a peak flow of 182 cfs was recorded at Canyon Lake Spillway (Station ID 841). No flows were recorded at the San Jacinto River at Ramona Expressway (Station ID 741).  A total of 1.06 to 2.07 inches of rainfall was recorded in the region during this storm (RCFCWCD 2019).

Mass loads for each chemical constituent at each location were calculated as the product of the event mean concentrations and the storm volumes for each storm event. The annual loads were calculated as the sum of the monitored event loads and the storm events where no sampling occurred, which are the product of the storm volumes for the storm events not monitored and the annual mean concentrations.

## San Jacinto River Watershed Annual Water Quality Summary

A summary of watershed water quality monitoring data for each of the four monitoring locations for the monitoring period of July 1, 2020 through June 30, 2021, is presented below. The complete set of analytical laboratory report results is included in Appendix A. Included with each summary of the monitoring data are the concentrations for each analyte. Also included are the estimated storm event loads and annual loads for each analyte.

### Summary of Monitoring Data – Salt Creek at Murrieta Road

Water quality samples were collected during two storm events at Salt Creek at Murrieta Road (Station ID 745) during the wet weather monitoring period from October 1, 2020 to May 31, 2021.

During the storm event on January 29, 2021 through January 31, 2021, a total of 33 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070465), flow for the storm event was estimated at 278 acre-feet or 91 million gallons (Mgal), which represents approximately 36% of the total annual flow.

During the storm event on March 10, 2021 through March 12, 2021, a total of 25 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070465), flow for the storm event was estimated at 168 acre-feet or 55 Mgal, which represents approximately 21% of the total annual flow.

Photos taken during the storm events are provided in Figure 2-7 and Figure 2-8.

Figure 2-7. Storm Event at Salt Creek at Murrieta Road (January 29-31, 2021)

Figure 2-8. Storm Event at Salt Creek at Murrieta Road (March 10-12, 2021)

Event and annual mean concentrations for each analyte are presented in Table 2-6.  Event and annual loads for each analyte are presented in Table 2-7.  Concentrations for nutrients for the two storm events ranged from 1.4 to 2.4 milligrams per liter (mg/L) for total nitrogen, and 0.25 to 0.53 mg/L for total phosphorus (Table 2-6).  Based on flow data provided by the nearby USGS stream gauge (Station ID 11070465), the total annual flow was estimated at 34,093,755 cubic feet (cf) or 255 Mgal for the period of July 1, 2020 through June 30, 2021. No dry weather flows enter Canyon Lake from Salt Creek at Murrieta Road (Station ID 745) so storm flows accounted for the total estimated annual load of nutrients. The estimated annual nutrient load was calculated to be 1,902 kg for total nitrogen and 396 kg for total phosphorus (Table 2-7) for the period of July 1, 2020 through June 30, 2021.

Table 2-6. Water Quality Concentrations at Salt Creek at Murrieta Road

| Analyte | Units | Event 1 | Event 2 | Annual Mean | Annual Geomean |
| --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | mg/L | 0.20 | 0.17 | 0.19 | 0.18 |
| Chemical Oxygen Demand | mg/L | 37.0 | 45.0 | 41.0 | 40.8 |
| Kjeldahl Nitrogen | mg/L | 1.80 | 0.92 | 1.36 | 1.29 |
| Nitrate as N | mg/L | 0.57 | 0.50 | 0.54 | 0.53 |
| Nitrite as N | mg/L | ND(<0.09) | (0.019)J | 0.00a | 0.00a |
| Organic Nitrogen | mg/L | 1.6 | 0.76 | 1.18 | 1.10 |
| Total Nitrogen | mg/L | 2.4 | 1.4 | 1.90 | 1.83 |
| Total Phosphorus | mg/L | 0.53 | 0.25 | 0.39 | 0.36 |
| Ortho Phosphate Phosphorus | mg/L | 0.21 | 0.16 | 0.19 | 0.18 |
| Total Dissolved Solids | mg/L | 440 | 240 | 340 | 325 |
| Total Hardness | mg/L | 170 | 89.5 | 130 | 123 |
| Total Suspended Solids | mg/L | 65.0 | 32.0 | 48.5 | 45.6 |

ND = not detected (analyte not detected at the indicated method detection limit (MDL)).

J- Reported value was detected above the MDL, but below the RL.

a - When a concentration was non-detect, the annual average value for compliance purposes was calculated by converting non-detect (ND) values to zero. If the result of the calculated mean was non-zero but below the corresponding MDL, the average value was reported as ND.

Table 2-7. Water Quality Event and Annual Loads at Salt Creek at Murrieta Road

| Analyte | Units | Load Event 1 | Load Event 2 | Annual Load |
| --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | kg | 69 | 35 | 181 |
| Chemical Oxygen Demand | kg | 12,688 | 9,302 | 39,038 |
| Kjeldahl Nitrogen | kg | 617 | 190 | 1,373 |
| Nitrate as N | kg | 195 | 103 | 521 |
| Nitrite as N | kg | 0a | 0a | 0a |
| Organic Nitrogen | kg | 549 | 157 | 1,196 |
| Total Nitrogen | kg | 823 | 289 | 1,902 |
| Total Phosphorus | kg | 182 | 52 | 396 |
| Ortho Phosphate Phosphorus | kg | 72 | 33 | 182 |
| Total Dissolved Solids | kg | 150,888 | 49,609 | 341,868 |
| Total Hardness | kg | 58,298 | 18,500 | 130,747 |
| Total Suspended Solids | kg | 22,290 | 6,615 | 49,071 |

a - When a concentration was non-detect, the annual load value for compliance purposes was calculated by converting non-detect (ND) values to zero.

Hydrographs with flow-weighted sample aliquot times are provided in Figure 2-9 and Figure 2-10. The figures were developed based on flow data provided by the nearby USGS stream gauge (Station ID 11070465).

Figure 2-9. Hydrograph of First Storm Event at Salt Creek at Murrieta Road
(January 29 -31, 2021)

Figure 2-10. Hydrograph of Second Storm Event at Salt Creek at Murrieta Road
(March 10 - 12, 2021)

### Summary of Monitoring Data – San Jacinto River at Goetz Road

Water quality samples were collected during two storm events at San Jacinto River at Goetz Road (Station ID 759) during the wet weather monitoring period from October 1, 2020 to May 31, 2021.

During the storm event on January 29, 2021 through January 31, 2021, a total of 29 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070365), flow for the storm event was estimated at 817 acre-feet or 266 Mgal, which represents approximately 51% of the total annual flow.

During the storm event on March 10, 2021 through March 12, 2021, a total of 21 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070365), flow for the storm event was estimated at 325 acre-feet or 106 Mgal, which represents approximately 20% of the total annual flow.

Photos taken during the storm events are provided in Figure 2-11 and Figure 2-12.

Figure 2-11. Storm Event at San Jacinto River at Goetz Road (January 29 - 31, 2021)

Figure 2-12. Storm Event at San Jacinto River at Goetz Road (March 10 - 12, 2021)

Event and annual mean concentrations for each analyte are presented in Table 2-8.  Event and annual loads for each analyte are presented in Table 2-9.  Concentrations for nutrients for the two storm events ranged from 1.8 to 2.0 mg/L for total nitrogen, and 0.40 to 0.56 mg/L for total phosphorus (Table 2-8).  Based on flow data provided by the nearby USGS stream gauge (Station ID 11070365), the total annual flow was estimated at 69,394,095cf or 519 Mgal for the period of July 1, 2020 through June 30, 2021.  No dry weather flows enter Canyon Lake from San Jacinto River at Goetz Road (Station ID 759) so storm flows accounted for the total estimated annual load of nutrients. The estimated annual nutrient load was calculated to be 3,794 kg for total nitrogen and 992 kg for total phosphorus (Table 2-9) for the period of July 1, 2020 through June 30, 2021.

Table 2-8. Water Quality Concentrations at San Jacinto River at Goetz Road

| Analyte | Units | Event 1 | Event 2 | Annual Mean | Annual Geomean |
| --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | mg/L | (0.08)J | 0.20 | 0.14 | 0.13 |
| Chemical Oxygen Demand | mg/L | 160 | 40 | 100 | 80.0 |
| Kjeldahl Nitrogen | mg/L | 1.4 | 0.93 | 1.17 | 1.14 |
| Nitrate as N | mg/L | 0.60 | 0.90 | 0.75 | 0.73 |
| Nitrite as N | mg/L | ND(<0.09) | (0.039)J | 0.02a | 0.00a |
| Organic Nitrogen | mg/L | 1.3 | 0.72 | 1.01 | 0.97 |
| Total Nitrogen | mg/L | 2.0 | 1.8 | 1.90 | 1.90 |
| Total Phosphorus | mg/L | 0.56 | 0.40 | 0.48 | 0.47 |
| Ortho Phosphate Phosphorus | mg/L | 0.17 | 0.22 | 0.20 | 0.19 |
| Total Dissolved Solids | mg/L | 120 | 170 | 145 | 143 |
| Total Hardness | mg/L | 85.0 | 74.3 | 79.7 | 79.5 |
| Total Suspended Solids | mg/L | 170 | 72 | 121 | 111 |

ND = not detected (analyte not detected at the indicated method detection limit (MDL)).

J- Reported value was detected above the MDL, but below the RL.

a - When a concentration was non-detect, the annual average value for compliance purposes was calculated by converting non-detect (ND) values to zero. If the result of the calculated mean was non-zero but below the corresponding MDL, the average value was reported as ND.

Table 2-9. Water Quality Event and Annual Loads at San Jacinto River at Goetz Road

| Analyte | Units | Load Event 1 | Load Event 2 | Annual Load |
| --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | kg | 81 | 80 | 239 |
| Chemical Oxygen Demand | kg | 161,230 | 16,035 | 232,912 |
| Kjeldahl Nitrogen | kg | 1,411 | 373 | 2,432 |
| Nitrate as N | kg | 605 | 361 | 1,383 |
| Nitrite as N | kg | 0a | 16 | 26 |
| Organic Nitrogen | kg | 1,310 | 289 | 2,161 |
| Total Nitrogen | kg | 2,015 | 722 | 3,794 |
| Total Phosphorus | kg | 564 | 160 | 992 |
| Ortho Phosphate Phosphorus | kg | 171 | 88 | 368 |
| Total Dissolved Solids | kg | 120,923 | 68,147 | 269,758 |
| Total Hardness | kg | 85,654 | 29,784 | 159,760 |
| Total Suspended Solids | kg | 171,307 | 28,862 | 267,502 |

a - When a concentration was non-detect, the annual load value for compliance purposes was calculated by converting non-detect (ND) values to zero.

Hydrographs with flow-weighted sample aliquot times is provided in Figure 2-13 and Figure 2-14.  The figure was developed based on flow data provided by the nearby USGS stream gauge (Station ID 11070365).

Figure 2-13. Hydrograph of First Storm Event at San Jacinto River at Goetz Road 
(January 29-31, 2021)

Figure 2-14. Hydrograph of Second Storm Event at San Jacinto River at Goetz Road 
(March 10 - 12, 2021)

### Summary of Monitoring Data – San Jacinto River at Ramona Expressway

Mystic Lake did not overflow during the wet weather monitoring period from October 1, 2020 to May 31, 2021. No flows were observed, and no samples were collected from the sampling station at San Jacinto River at Ramona Expressway (Station ID 741) during the 2020-2021 monitoring year.

### Summary of Monitoring Data – Canyon Lake Spillway

Water quality samples were collected during two storm events at Canyon Lake Spillway (Station ID 841) during the wet weather monitoring period from October 1, 2020 to May 31, 2021.

During the storm event on January 29, 2021 through February 1, 2021, a total of 31 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070500), flow for the storm event was estimated at 1,142 acre-feet or 372 Mgal, which represents approximately 42% of the total annual inflow to Lake Elsinore from Canyon Lake.

During the storm event on March 10, 2021 through March 15, 2021, a total of 42 discrete samples were collected across the hydrograph at two-hour intervals and a single flow-weighted composite sample was submitted for analysis. Based on data provided by the nearby USGS stream gauge (Station ID 11070500), flow for the storm event was estimated at 1,192 acre-feet or 388 Mgal, which represents approximately 44% of the total annual inflow to Lake Elsinore from Canyon Lake.

The flows from Canyon Lake do not include runoff from the local surrounding watershed into Lake Elsinore. Photos taken during the storm events are provided in Figure 2-15 and Figure 2-16.

Figure 2-15. Storm Event Sampling Below the Canyon Lake Spillway (March 10 - 15, 2021)

Figure 2-16. Canyon Lake Spillway (March 10 - 15, 2021)

Event and annual mean concentrations of each analyte are presented in Table 2-10. Event and annual loads for each analyte are presented in Table 2-11. Concentrations of nutrients for the two storm events ranged from 1.3 to 2.1 mg/L for total nitrogen, and 0.036 to 0.07 mg/L for total phosphorus (Table 2-10). Based on flow data provided by the nearby USGS stream gauge (Station ID 11070500), the total annual flow was estimated at 117,374,904 cf or 878 Mgal for the period of July 1, 2020 through June 30, 2021. The USGS stream gauge (Station ID 11070500) located downstream of the Canyon Lake Spillway (Station ID 841) sampling location has minimal dry weather flow and storm flows account for the vast majority of the estimated annual load of nutrients exiting Canyon Lake. The estimated annual nutrient load was calculated to be 5,626 kg for total nitrogen and 175 kg for total phosphorus (Table 2-11) for the period of July 1, 2020 through June 30, 2021.

Table 2-10. Water Quality Concentrations at Canyon Lake Spillway

| Analyte | Units | Event 1 | Event 2 | Annual Mean | Annual Geomean |
| --- | --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | mg/L | 0.50 | 0.28 | 0.390 | 0.37 |
| Chemical Oxygen Demand | mg/L | 37.0 | 19.0 | 28.0 | 26.5 |
| Kjeldahl Nitrogen | mg/L | 1.7 | 0.9 | 1.30 | 1.24 |
| Nitrate as N | mg/L | 0.41 | 0.38 | 0.40 | 0.39 |
| Nitrite as N | mg/L | ND(<0.09) | (0.023)J | 0.012a | 0.00a |
| Organic Nitrogen | mg/L | 1.2 | 0.62 | 0.90 | 0.86 |
| Total Nitrogen | mg/L | 2.1 | 1.3 | 1.70 | 1.65 |
| Total Phosphorus | mg/L | 0.07 | 0.036 | 0.05 | 0.05 |
| Ortho Phosphate Phosphorus | mg/L | ND(<0.030) | 0.018 | 0.009a | 0.00a |
| Total Dissolved Solids | mg/L | 430 | 490 | 460 | 459 |
| Total Hardness | mg/L | 230 | 233 | 232 | 232 |
| Total Suspended Solids | mg/L | 2.0 | ND(<5.0) | 1.0 | 0.0 |

ND = not detected (analyte not detected at the indicated method detection limit (MDL)).

J- Reported value was detected above the MDL, but below the RL.

a - When a concentration was non-detect, the annual average value for compliance purposes was calculated by converting non-detect (ND) values to zero. If the result of the calculated mean was non-zero but below the corresponding MDL, the average value was reported as ND.

Table 2-11. Water Quality Event and Annual Loads at Canyon Lake Spillway

| Analyte | Units | Load Event 1 | Load Event 2 | Annual Load |
| --- | --- | --- | --- | --- |
| Ammonia-Nitrogen | kg | 705 | 412 | 1,289 |
| Chemical Oxygen Demand | kg | 52,138 | 27,946 | 92,508 |
| Kjeldahl Nitrogen | kg | 2,396 | 1,324 | 4,296 |
| Nitrate as N | kg | 578 | 559 | 1,312 |
| Nitrite as N | kg | 0a | 34 | 39 |
| Organic Nitrogen | kg | 1,691 | 912 | 3,007 |
| Total Nitrogen | kg | 2,959 | 1,912 | 5,626 |
| Total Phosphorus | kg | 99 | 53 | 175 |
| Ortho Phosphate Phosphorus | kg | 0a | 26 | 30 |
| Total Dissolved Solids | kg | 605,925 | 720,701 | 1,530,747 |
| Total Hardness | kg | 324,099 | 342,701 | 769,526 |
| Total Suspended Solids | kg | 2,818 | - | 3,262 |

a - When a concentration was non-detect, the annual load value for compliance purposes was calculated by converting non-detect (ND) values to zero.

Hydrographs with flow-weighted sample aliquot time are provided in Figure 2-17 and Figure 2-18. The figure was developed based on flow data provided by the nearby USGS stream gauge (Station ID 11070365). A hydrograph of the Canyon Lake Level at Railroad Canyon Dam Spillway compared to the spillway elevation is provided in Figure 2-19.

Figure 2-17. Hydrograph of First Storm Event at Canyon Lake Spillway
(January 29-February 1, 2021)

Figure 2-18. Hydrograph of Second Storm Event at Canyon Lake Spillway
(March 10-15, 2021)

Figure 2-19. Canyon Lake Level at Railroad Canyon Dam Spillway

## San Jacinto River Watershed Rainfall Records

The RCFC&WCD maintains rainfall records for rain gauges located within or near the San Jacinto River Watershed as shown in Table 2-12.

Table 2-12. San Jacinto River Watershed Rainfall Gauges

| Station ID | Station Description | Latitude | Longitude | Elevation (ft.) |
| --- | --- | --- | --- | --- |
| 67 | Lake Elsinore | 33.668712 | -117.332380 | 1281 |
| 152 | Perris | 33.786980 | -117.231831 | 1494 |
| 155 | Perris / Moreno Valley – Pigeon Pass | 33.987703 | -117.270221 | 1902 |
| 186 | Hemet / San Jacinto | 33.787067 | -116.959024 | 1554 |
| 248 | Winchester | 33.702903 | -117.090382 | 1466 |

Rainfall data recorded at these five stations for the period July 1, 2020, through June 30, 2021, are summarized in Table 2-13.

| Table 2-13. Summary Rainfall Data (July 2020 to June 2021) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Monthly Rainfall (inches) | Lake Elsinore | Perris CDF | Pigeon Pass | Hemet / San Jacinto | Winchester |
| Jul | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Aug | 0.00 | 0.00 | 0.00 | 0.04 | 0.00 |
| Sep | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Oct | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Nov | 0.35 | 0.10 | 0.30 | 0.37 | 0.20 |
| Dec | 1.08 | 0.88 | 1.59 | 1.50 | 1.31 |
| Jan | 1.52 | 1.71 | 2.42 | 2.06 | 1.76 |
| Feb | 0.04 | 0.04 | 0.02 | 0.12 | 0.07 |
| Mar | 1.30 | 1.77 | 2.35 | 1.30 | 1.56 |
| Apr | 0.00 | 0.02 | 0.02 | 0.09 | 0.25 |
| May | 0.00 | 0.00 | 0.02 | 0.01 | 0.00 |
| Jun | 0.01 | 0.00 | 0.09 | 0.02 | 0.16 |
| Annual Rainfall (inches) | 4.30 | 4.53 | 6.81 | 5.50 | 5.31 |

# In-Lake Monitoring

## Background

Routine in-lake monitoring was initiated in 2006 by local stakeholders in cooperation with the RWQCB at three open water locations in Lake Elsinore and four locations in Canyon Lake.  Initially, monitoring consisted of monthly sampling October to May, and biweekly sampling June to September, with grab samples collected at the surface, within the water column, and/or as depth-integrated samples (depending on the lake and the analyte).  Based on modifications adopted to the sampling program (RWQCB Resolution No. R8-2011-0023), in 2011-2012 sampling locations in Lake Elsinore and Canyon Lake were reduced to one and three stations, respectively, for analytical chemistry. This decision was based on a review of available data that indicated consistent similar nutrient concentrations and physical water quality parameters among the three sampling sites in Lake Elsinore and two sites in the East Basin of Canyon Lake.  This cost savings allowed for shifting resources toward several implementation strategies aimed at reducing nutrient impacts in both lakes as described in RWQCB Resolution No. R8-2011-0023.  All in-lake monitoring was then suspended temporarily during the 2013-2014 and 2014-2015 FYs to further redirect resources toward implementing in-lake best management practices.  Starting in FY 2015-2016, ongoing in-lake sampling was resumed and is required to estimate progress toward attaining nutrient TMDL targets and calculating annual and 10-year running averages. The following sections describe monitoring methods and results in both lakes for the 2020-2021 FY.

## Lake Elsinore Monitoring

### Sampling Station Locations and Frequency

To maintain consistency and facilitate the assessment of trends toward meeting compliance goals, the in-lake monitoring design was resumed in July 2015 using the three former stations outlined in the approved Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan (LESJWA, 2006; Figure 3-1, Table 3-1).  Analytical chemistry samples and in-situ water quality profile readings were collected at Site LE02, while only in-situ water quality profile readings were performed at the remaining two stations (LE01 and LE03). Profile readings for all three stations were taken in both the morning and afternoon. Water chemistry samples collected at Site LE02 were analyzed for those constituents outlined in Table 3-2.  Sampling in Lake Elsinore was conducted monthly during summer months (June-September) and bi-monthly (i.e., every other month) for the remainder of the monitoring year, for a total of eight sampling events per year. In-lake TMDL sampling events were coordinated to correspond with satellite overpass dates to facilitate the comparison of in-lake and satellite derived chlorophyll-a data (see Section 3.4).

Figure 3-1. Lake Elsinore Sampling Locations

Table 3-1.  Lake Elsinore TMDL Monitoring Locations

| Site | Latitude | Longitude |
| --- | --- | --- |
| LE01 | 33.668978° | -117.364185° |
| LE02 | 33.663344° | -117.354213° |
| LE03 | 33.654939° | -117.341653° |

Table 3-2.  2020-2021 In-lake Analytical Constituents and Methods for Lake Elsinore

| Parameter | Analysis Method | Sampling Method |
| --- | --- | --- |
| Analytical Chemistry |  |  |
| Nitrite Nitrogen (NO2-N) | EPA 353.2 | Depth Integrated |
| Nitrate Nitrogen (NO3-N) | EPA 353.2 | Depth Integrated |
| Total Kjeldahl Nitrogen (TKN) | EPA 351.2 | Depth Integrated |
| Total Nitrogen (TN)1 | Calculated | Depth Integrated |
| Ammonia Nitrogen (NH4-N) | EPA 350.1 | Depth Integrated |
| Sulfide | SM 4500S2 D | Depth Integrated |
| Total Phosphorus (TP) | EPA 365.3 | Depth Integrated |
| Soluble Reactive Phosphorus (SRP / Ortho-P) | EPA 365.3, EPA 353.2 | Depth Integrated |
| Chlorophyll-a | SM 10200H | Surface (0-2m) & Depth Integrated |
| Total Dissolved Solids (TDS) | SM 2540 C | Depth Integrated |

US EPA - United States Environmental Protection Agency; m- meter; SM- standard method1 Total Nitrogen calculated as TKN+NO2+NO3

### Sampling Methods

Depth-integrated composite samples for analytical chemistry were collected at Site LE02 by utilizing a peristaltic pump and lowering/raising an inlet tube through the water column at a uniform speed, creating a composite sample of the entire water column. Two samples were collected for chlorophyll-a: 1) a full depth-integrated composite sample as described above; and 2) a 0-2-meter (m) depth-integrated composite surface sample. All samples for chemical analysis were placed and held on wet ice immediately following collection and transferred to a local courier or shipping company on the same day of collection. Samples for analysis of nutrients, ammonia, sulfide, TDS, and chlorophyll-a were submitted to Weck Laboratories Inc., located in City of Industry, California.

Secchi disk readings for water clarity, as well as in-situ water column profile data, were typically recorded between 7:00 and 9:00 in the morning at all three Lake Elsinore stations using pre-calibrated hand-held YSI field meters or equivalent for pH, temperature, DO, and specific conductivity at 1-m intervals throughout the water column.  This data was used to assess lateral and vertical spatial variability within the lake.  End-of-the-day water column profiles (i.e., after ~2:00pm) were also recorded for the same in-situ parameters at all three stations to assess any potential temporal variability in these parameters over the course of a day.

Satellite imagery was used as a tool to remotely measure chlorophyll-a and turbidity concentrations at the water surface.  These images provide a more complete picture of spatial variability that can exist for these two parameters at any given point in time. In-lake sampling dates were selected to correspond with satellite overpasses to enable comparison of analytical laboratory and satellite derived chlorophyll-a concentrations.  Processed satellite imagery and associated reports were provided by EOMAP GmbH & co. KG (EOMAP) based in Germany (Castle Seefeld Schlosshof).  Satellite imagery was also used to estimate the likelihood of a harmful algae bloom.

### Water Quality Summary

A summary of the in-lake monitoring events for Lake Elsinore for the period of July 1, 2020 to June 30, 2021 is presented below.  A total of eight Lake Elsinore events were sampled during this period under the TMDL monitoring program, with five occurring in 2020 (July 28, August 13, September 14, October 5 and December 9) and three in 2021 (February 17, April 8 and June 2).  Complete monthly water quality profiles can be found in Appendix B.  Detailed analytical chemistry lab reports for each event are contained in Appendix C.  Satellite imagery reports for each event are provided in Appendix D.  Current data in the context of historical water quality monitoring results from 2002-present are presented in Appendix E.

A summary of mean water column profile values for each site and monitoring event are presented in Tables 3-3 and 3-4.  Water column mean profile statistics for each site across the entire monitoring period are presented in Table 3-5.  Mean values for water column measurements for each site, as well as the lake-wide mean are also summarized graphically in Figures 3-2 through 3-7.  The measurements during the morning and afternoon of any given monitoring event were averaged prior to summarizing in the tables and figures below.

Table 3-3. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2020 Monthly Means for Each Site (July – Dec 2020)

Notes:

°C = degrees Celsius; µS/cm = microSiemens per centimeter; m= meter; mg/L = milligrams per liter

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in bottom 1m of sampling site

Bold Underline - Indicates exceedance of 2020 TMDL target

Table 3-4. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2021 Monthly Means for Each Site (February – June 2021)

Notes:

°C = degrees Celsius; µS/cm = microSiemens per centimeter; m= meter; mg/L = milligrams per liter

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in bottom 1m of sampling site

Bold Underline - Indicates exceedance of 2020 TMDL target

Regular differences in morning and afternoon water column mean measurements were noted across all monitoring events, with consistent increases in pH (up to 0.1 standard units (SU)), DO (up to 3.3 mg/L), and temperature (up to 1.3 °C increase), while specific conductivity remained stable. These increases were more prominent in summer months, while little deviation was observed during winter and spring monitoring events.

Temperature exhibited a typical pattern with lowest values occurring during the winter events (December and February) and highest values in summer months (July and August).  Historical Lake Elsinore data shows that the average lake-wide annual temperature during this monitoring year was the highest observed over the last 19 years of record when including data from complete monitoring years July to June (i.e., temperature data available for each month of the monitoring year).  Generally, the greatest DO concentrations throughout the water column (both water column mean and 1-meter from bottom) were observed in February at all three sites. One exception to this was the highest water column mean for LE03 observed in June 2021. Concentrations of DO near the bottom, while lower, generally tracked with the overall water column mean for all three sites.  While all three sites exhibited a divergence in water column mean and 1-meter from bottom DO beginning in April 2021, the 1-m from the bottom DO reading dropped dramatically at Site LE02. Site LE02 is located in the deepest portion of the lake and would be expected to have a larger hypolimnion with low DO than other portions of the lake.  These diverging measurements indicate that the lake was starting to stratify, further supported by the concurrent increased temperatures recorded during the April and June 2021 events and historical trends that demonstrate stratification in portions of the lake beginning during this period.  The 12-month rolling mean water column DO concentration at Site LE02 was above the 2015 TMDL target of 5.0 mg/L for the first event of the monitoring year, and just below the target for the remainder of the events (Figure 3-2).  The rolling mean DO concentration 1-m above the lake bottom at Site LE02 remained below the 2020 TMDL target of 5.0 mg/L for the entire monitoring year, ranging from 2.1 to 3.5 mg/L.

Conductivity exhibited a gradual increased from July through December 2020, from approximately 3144 to 3474 microSiemens per centimeter (µS/cm). The conductivity dropped slightly between December 2020 and February 2021, as a series of storms moved through the region.  Conductivity then remained steady before an increase in June 2021 to near its highest level of the monitoring year.

Relative to the previous 2 monitoring years (2018-2019 and 2019-2020), the 2020-2021 pH values exhibited a much different annual pattern.  The pH measurements from previous 2 years remained relatively steady across the monitoring year and within a relatively narrow band.  During the 2020-2021 monitoring year, a notable drop of 1.0 SU was observed during the 2nd and 3rd field events (August and September), followed by a gradual increase.   A similar drop was observed in August of the 2017-2018 monitoring year.  There is no obvious reason for the decrease in pH across these two monitoring events but could be partially due to the algal composition of the lake as a result of the known close relationship between the processes involved in algal photosynthesis and pH levels particularly in lakes with high algal biomass.

Water clarity measured using a Secchi disk increased gradually at LE02 across the summer, fall and early winter months, from 0.8 feet (ft) in July to 1.1 ft in February (Figure 3-7).  This pattern is somewhat different from previous monitoring years which generally exhibit a decrease in water clarity across the summer and fall months. A decrease in clarity was then observed between February and June.  The first overflow of the Canyon Lake spillway was February 1, with the majority of the precipitation in this monitoring year falling between January and March.  While not a tight relationship, the Secchi depths observed generally exhibited an inverse relationship with algal density (i.e. chlorophyll-a concentrations) at Site LE02.  Sites LE01 and LE03 both displayed similar Secchi depths and patterns as those exhibited at Site LE02.

For further comparisons regarding in-situ water quality parameters, Table 3-5 includes lake-wide averages observed for the current 2020-21 monitoring year, as well as the prior 2018-19 and 2019-20 monitoring years.

Table 3-5. In-Situ Water Quality Parameter Measurements in Lake Elsinore – 2020-2021 Annual Mean Statistics for Each Site

Notes:

°C = degrees Celsius; µS/cm = microSiemens per centimeter; m= meter; mg/L = milligrams per liter

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in bottom 1m of sampling site

Bold Underline - Indicates exceedance of 2020 TMDL target

Figure 3-2. Water Column Mean Dissolved Oxygen (DO) Rolling Average – Lake Elsinore for Site LE02

Each data point is calculated by averaging the measurement from each event with the previous seven events (i.e., one year of data) to obtain a rolling average. Therefore, the graph represents data collected from August 2019 to June 2021.

Figure 3-3. In-Situ Physical Water Quality Parameters - Lake Elsinore - Site LE01

Figure 3-4. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE02

Figure 3-5. In- Situ Physical Water Quality Parameters - Lake Elsinore Site LE03

Figure 3-6. Monthly Lake-wide Mean of In-Situ Physical Water Quality Parameters – Mean of All Three Stations

Figure 3-7. In- Situ Water Clarity Using a Secchi Disk - Lake Elsinore Site LE02

#### Analytical Chemistry

Monthly analytical results and annual summary chemistry concentrations at Site LE02 are presented in Tables 3-6 and 3-7, respectively.  Concentrations of analytes at Site LE02 are graphically presented in Figures 3-8 through 3-10.

Total nitrogen concentrations were steady across the summer, fall and winter months, before decreasing in April 2021 (Figure 3-8), likely as a result of the largest precipitation events coming after the February collection event and the overflow of the Canyon Lake spillway. Total nitrogen values across the monitoring year ranged from 4.6 mg/L in July and September 2020 dropping to 2.6 mg/L in April 2021. The annual mean concentration of total nitrogen was 4.1 mg/L (4.4 mg/L in the previous monitoring year). The total nitrogen rolling average concentration, calculated by averaging the measurement from each event with the previous seven events (i.e., one year of data), exceeded the current 2020 TMDL target of 0.75 mg/L for each event (Figure 3-9).

Total phosphorus concentrations ranged from 0.13 to 0.28 mg/L across all monitoring events.  The lowest total phosphorus concentration was observed in July (0.13 mg/L).  The remainder of the monitoring events exhibited higher concentrations of total phosphorus, all within a narrow range (0.25 – 0.28 mg/L; Figure 3-8).  Dissolved oxygen concentrations in July and August 2020 indicated that the lake was stratified during both events and didn’t de-stratify until sometime between the October and December 2020 monitoring events.  Typically, total phosphorus concentrations exhibit an increase following destratification as phosphorus that had fluxed from the sediment during the stratification period as a result of low dissolved oxygen concentration near the sediment surface, is dispersed throughout the water column. So, the jump in total phosphorus from July to August 2020, and the lack of increase in depth-integrated total phosphorus when the lake did de-stratify is not clear but could be related to a range of factors including greater vertical mixing with bottom waters and physical disturbance of the sediments.  All QA data associated with the July 2020 total phosphorus laboratory data meets acceptability criteria and does not explain an anomalously low value for July.   The annual mean concentration of total phosphorus was 0.24 mg/L, up from the 0.18 mg/L annual mean from the previous monitoring year.  The total phosphorus rolling average concentration exceeded the current 2020 TMDL target of 0.1 mg/L for each event (Figure 3-9).

Total ammonia-N concentrations while quite variable, exhibited an increase during the first two monitoring events followed by a general downward trend through the remainder of the monitoring year, with an annual mean of 0.31 mg/L.  Total ammonia concentrations of 0.40 and 0.69 mg/L in August and September 2020 exceeded their corresponding Criterion Continuous Concentration (CCC) objectives (Table 3-6). No other samples exceeded the total ammonia CCC or the Criterion Maximum Concentration (CMC) objective. The initial increase in total ammonia-N during the first two monitoring events is likely the result of the lake stratification and low DO near the bottom, as these two months showed the strongest differentiation in DO concentrations between the water column and 1-m from the bottom.  This low DO near the sediment can facilitate the flux of ammonia.

Total dissolved solids (TDS) concentrations showed a slight increase from July to October (1800 to 2000 mg/L) and then remained stable through the remainder of the monitoring year until a slight increase again in June 2021.  This pattern follows that observed for specific conductivity (Figure 3-6).  The typical decrease of TDS in Lake Elsinore due to winter storm events was not observed this year, likely due to the reduced precipitation totals of this past winter, and the paucity of water coming over the Canyon Lake spillway.

Depth-integrated concentrations of chlorophyll-a across all eight sampling events ranged from 61 to 252 µg/L.  Surface (0-2m) chlorophyll-a concentrations ranged from 55 to 200 µg/L. Surface and depth-integrated samples generally tracked with each other.  Chlorophyll-a concentrations exhibited a general decrease across the summer and early fall months (Figure 3-10).  This is unusual for this lake which typically exhibits an increase in chlorophyll-a during these months until winter storms bring rain to the area.  This was followed by an increase in chlorophyll-a concentration from 61 to 150 µg/L between April and June 2021.  The mean chlorophyll-a concentration observed in samples collected during the summer months (June 2020 through September 2020) was 212 µg/L for depth-integrated samples and 179 µg/L for surface samples.  The August (depth-integrated) and September (depth-integrated and surface) chlorophyll-a samples for Lake Elsinore were not analyzed due to a laboratory error.  The chlorophyll was extracted from the glass-fiber filters, but the vials holding the extract material were dropped and broken, unable to be recovered.

Table 3-6.  Monthly Analytical Chemistry Results for Lake Elsinore in 2020-2021

Notes:

When a concentration was non-detect (ND), the annual mean value for compliance purposes was calculated by converting ND values to zero. If the result of the calculated mean was non-zero, but below the corresponding MDL, the mean value was reported as ND.

a - Total Nitrogen = TKN+NO2+NO3

b - The concentration of unionized ammonia was calculated using equation by Thursby (1986), based on site specific pH and temperature recorded at each location.

c – Sample measured out of holding time

ND – Not detected; NA – Not Applicable/ available; NM:LE – Not measured due to laboratory error

DI = Depth integrated; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; MDL – method detection limit; RL – reporting limit; J - Reported value is an estimate as detection was above the MDL, but below the RL

Bold Underline - Indicates exceedance of TMDL or Basin Plan target

* Exceeds 2004 TMDL Permit NH3 CCC; ** Exceeds 2004 TMDL Permit NH3 CMC

Table 3-7. Analytical Chemistry Summary for Lake Elsinore – Annual Mean Statistics (2020-2021)

Notes:

When a concentration was non-detect, the annual value for compliance purposes was calculated by converting non-detect (ND) values to zero. If the result of the calculation was below the corresponding MDL, the average value was reported as ND.

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Summer average (June 2020 – September 2020)

d - Values calculated using water column mean ammonia, temperature, salinity and pH.  Calculated using equation by Thursby (1986). The range of TMDL target thresholds apply to individual samples, not applicable to annual means.

e - Measured outside of holding time

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

4 - Based on the 2013 EPA Aquatic Life Ambient Water Quality Criteria for Ammonia - Freshwater 2013 (EPA 822-R-18-002). The CMC calculation is assuming the absence of Oncorhynchus spp and is provided for comparison only.

NA – Not applicable/ available; ND – not detected

DI = Depth integrated; Surf = Surface 0-2m

mg/L – micrograms per liter; ug/L – milligrams per liter; MDL – method detection limit; RL – reporting limit; J –Reported value was detected above the MDL, but below the RL

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

* Exceeds 2004 TMDL Permit NH3 CCC; ** Exceeds 2004 TMDL Permit NH3 CMC

Figure 3-8. Lake Elsinore Analytical Chemistry – Depth-Integrated Means at Site LE02
(July 2020-June 2021)

Long term trends can be found in Appendix E

Figure 3-9. Lake Elsinore Analytical Chemistry – Total Nitrogen and Phosphorus Rolling Averages (July 2020 – June 2021)

Each data point is calculated by averaging the value of each event with the previous seven events (i.e. one year of data) to obtain a rolling average. Therefore, the graph represents data collected from August 2019 to June 2021.

Figure 3-10. Lake Elsinore Analytical Chemistry – Depth-Integrated and Surface Chlorophyll-a at Site LE02

August (Depth-integrated) and September (Depth-integrated & Surface) samples not analyzed due to lab error (see text). Long term trends can be found in Appendix E

## Canyon Lake Monitoring

### Sampling Station Locations and Frequency

Similar to Lake Elsinore, sampling parameters and locations in Canyon Lake were based on the TMDL monitoring conducted between 2006 and 2012 to provide consistency in assessing trends toward meeting compliance goals.  The in-lake monitoring design halted in 2012 and resumed in July 2015 using the four stations outlined in the approved Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan (LESJWA, 2006; Figure 3-11, Table 3-8). Two sites are located in the main body of the lake (CL07 near the dam and CL08 in the northern arm), and two in the East Bay (CL09 and CL10).  Samples for analytical chemistry and chlorophyll-a were collected at all four sites, in addition to morning and afternoon in-situ water quality profile readings.

Sampling in Canyon Lake was conducted bi-monthly (i.e., every other month) concurrent with the TMDL sampling in Lake Elsinore and was also coordinated with satellite overpass dates (see Section 3.4).

Table 3-8.  Canyon Lake TMDL Monitoring Locations

| Site | Latitude | Longitude |
| --- | --- | --- |
| CL07 | 33.678027° | -117.275135° |
| CL08 | 33.688211° | -117.268944° |
| CL09 | 33.681100° | -117.258892° |
| CL10 | 33.679495° | -117.250669° |

Figure 3-11. Canyon Lake Sampling Locations

### Sampling Methods

Samples for analytical chemistry were collected in the same manner as in Lake Elsinore using a peristaltic pump to collect depth-integrated composite samples. Two samples were collected for chlorophyll-a: 1) a full depth-integrated composite sample; and 2) a 0-2-m depth-integrated composite surface sample. All analytical samples were held on wet ice immediately following collection and transferred to a local courier or shipping company on the same day of collection. Samples for analysis of nutrients, ammonia, sulfide, TDS, total suspended solids and chlorophyll-a were submitted to Weck Laboratories Inc., located in City of Industry, California (Table 3-9).

Beginning with the February 2017 sampling event, the TMDL Task Force directed that the pre- and post-alum application monitoring be integrated into the routine TMDL monitoring, given that the monitored analytes were largely identical to the TMDL monitoring, with the exception of aluminum and total suspended solids.  Given this directive, total/dissolved aluminum and total suspended solids were added to the nutrient TMDL monitoring analyte list for all subsequent routine TMDL monitoring events on Canyon Lake. During the 2020-2021 monitoring period, Canyon Lake alum applications were performed during the week of October 12, 2020.  Pre-alum application monitoring events were performed on October 5, 2020, with the subsequent respective bi-monthly TMDL event on December 9, 2020 serving as the post-alum application monitoring.

In-situ water column profile data was recorded in the morning at all four Canyon Lake stations using pre-calibrated hand-held YSI field meters or equivalent for pH, temperature, DO, and specific conductivity at 1-m intervals throughout the water column.  These data were used to assess lateral and vertical spatial variability within the lake.  End-of-the-day water column profiles (i.e., after ~2:00pm) were also recorded for the same suite of in-situ parameters at all stations to assess any potential temporal variability in these parameters over the course of a day.  Water clarity was also assessed with a Secchi disk at all stations.

Satellite imagery was used to remotely measure chlorophyll-a and turbidity concentrations at the water surface in Canyon Lake.  Satellite imagery was also used to estimate the likelihood of a harmful algae bloom.

Table 3-9.   In-lake Analytical Constituents and Methods for Canyon Lake (2020-2021)

| Parameter | Analysis SOP # | Sampling Method |
| --- | --- | --- |
| Analytical Chemistry |  |  |
| Nitrite Nitrogen (NO2-N) | EPA 353.2 | Depth Integrated |
| Nitrate Nitrogen (NO3-N) | EPA 353.2 | Depth Integrated |
| Total Kjeldahl Nitrogen (TKN) | EPA 351.2 | Depth Integrated |
| Total Nitrogen (TN) | Calculated | Depth Integrated |
| Ammonia Nitrogen (NH4-N) | EPA 350.1 | Depth Integrated |
| Sulfide | SM 4500S2 D | Depth Integrated |
| Total Phosphorus (TP) | EPA 365.3 | Depth Integrated |
| Soluble Reactive Phosphorus (SRP / Ortho-P) | EPA 365.3 | Depth Integrated |
| Chlorophyll-a | SM 10200H | Surface (0-2m) & Depth Integrated |
| Total Dissolved Solids (TDS) | SM 2540 C | Depth Integrated |
| Total Suspended Solids (TSS) | SM 2540D | Depth Integrated |
| Total Aluminum | EPA 200.7 | Depth Integrated |
| Dissolved Aluminum | EPA 200.7 | Depth Integrated |

Notes:

US EPA - United States Environmental Protection Agency; m- meter; SM- standard method

### Water Quality Summary

A summary of the in-lake monitoring events for Canyon Lake for the period of July 1, 2020 to June 30, 2021 is presented below.  A total of six events were sampled under the TMDL monitoring program, with three occurring in 2020 (August 13, October 5 and December 9) and three in 2021 (February 17, April 8 and June 2).  Complete monthly water quality profiles can be found in Appendix B.  Detailed analytical chemistry lab reports for each event are contained in Appendix C.  Satellite imagery reports for each event are provided in Appendix D.  Current data in the context of historical water quality monitoring results from 2002-present are presented in Appendix E.

#### Water Column Profiles

A summary of water column profile mean values for each site and monitoring event are presented in Tables 3-10 and 3-11.  A summary of water column profile mean values for each basin (i.e., Main Lake and Eastern) are presented in Tables 3-12 and 3-13. Water column profile mean statistics for each site across the entire monitoring period are presented in Table 3-14.  Mean water column values across the annual cycle are also summarized graphically in Figures 3-12 to 3-16.

For the purposes of this report, the epilimnion is defined as the region of the water column above the thermocline, while the hypolimnion is the region of the water column below the thermocline, with both regions exhibiting relatively stable temperatures.  The thermocline portion of the water column is defined as the region between the epilimnion and hypolimnion where a marked drop in temperature per unit of depth is evident (i.e., >1.0°C over 1-m depth differential). Measurements within the thermocline were excluded from epilimnion and hypolimnion averaging. Full water column means included data recorded from all three zones, if present.

For both the Main Basin and East Basin, temperature exhibited a typical pattern with the lowest values occurring during the winter months (December and February) and highest values in summer months (August).  Over the past 14 years of available records when including data from complete monitoring years July to June (i.e., temperature data available for each month of the monitoring year), Canyon Lake data shows that a different pattern than that of Lake Elsinore.  The highest lake-wide mean temperature was observed in the 2011-12 and 2016-17 monitoring years (19.7°C), compared to 18.9°C for the current year. Dissolved oxygen concentrations for both basins reflected an inverse pattern with temperature, exhibiting elevated concentrations during the winter months when averaged throughout the water column, reaching a maximum concentration in February 2021. When the thermocline develops in the lake, typically beginning in late spring through early fall period as the surface water heats up, DO concentrations within the epilimnion and hypolimnion diverge, with hypolimnion concentrations falling substantially during that timeframe.  This same pattern was observed this monitoring year with the lake exhibiting stratification in August and October 2020, being de-stratified in December and February, and becoming stratified again in April 2021 (Figures 3-12 and 3-13).  The rolling annual average DO concentration within the epilimnion was greater than the current 2015 TMDL target of 5.0 mg/L for all five events in both Basins when stratification was present (Figure 3-14). However, the rolling average DO concentration was never above the 2020 TMDL target of 5.0 mg/L in the hypolimnion.  The rolling average of the full water column mean was above 5.0 mg/L for all monitoring dates (Figure 3-15).

Conductivity within the epilimnion and hypolimnion (when present) and the water column in general exhibited a gradual increase over the monitoring period.  Average specific conductivity throughout the entire water column in the Main Basin of Canyon Lake (mean of CL07 & CL08) rose from 677 to 868 µS/cm (Tables 3-12 and 3-13, Figure 3-12).  Locations in the East Basin (mean of CL09 & CL10) exhibited a similar increase across the monitoring year from 804 to 1051 µS/cm. Mean values for pH were slightly higher in the Eastern Basin than the Main Basin, with values ranging from 7.57 – 8.58 and 7.33 – 7.94, respectively.  Values for pH within the epilimnion and hypolimnion tended to diverge as the thermocline developed, with pH values remaining higher in the epilimnion.

Secchi depths remained stable and relatively similar for both portions of the lake through December 2020.  The Main Body then exhibited a notable increase in secchi depth (increased water clarity) in February and April 2021.  This same increase was observed in the East Basin but was delayed by one monitoring event (Figure 3-16).  There appears to be an inverse relationship with chlorophyll-a in the Main Basin, however this is not the case with the East Bain.

For further comparisons regarding in-situ water quality parameters, Table 3-14 includes lake-wide averages observed for the current 2020-21 monitoring year, as well as the prior 2018-19 and 2019-20 monitoring years.

Table 3-10. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (August – December 2020)

Notes:

Epi = epilimnion; Hypo = hypolimnion; -- not applicable due to lack of thermocline

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in the hypolimnion

Bold Underline - Indicates exceedance of TMDL target

Table 3-11. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Site (February – June 2021)

Notes:

Epi = epilimnion; Hypo = hypolimnion; -- not applicable due to lack of thermocline

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in the hypolimnion

Bold Underline - Indicates exceedance of TMDL target

Table 3-12. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (August – December 2020)

Notes:

Epi = epilimnion; Hypo = hypolimnion; -- not applicable due to lack of thermocline

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in the hypolimnion

Bold Underline - Indicates exceedance of TMDL target

Table 3-13. In-Situ Water Quality Parameter Measurements for Canyon Lake - Monthly Means for Each Basin (February – June 2021)

Notes:

Epi = epilimnion; Hypo = hypolimnion; -- not applicable due to lack of thermocline

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in the hypolimnion

Bold Underline - Indicates exceedance of TMDL target

Table 3-14. In-Situ Water Quality Parameter Measurements for Canyon Lake - Annual Mean Statistics for Each Site (August 2020 – June 2021) and Comparison to Previous Monitoring Events

Notes:

-- not applicable due to lack of thermoclineValues reported for epilimnion and hypolimnion are the arithmetic mean of measurements collected across all months sampled in which stratification was present.

Main Basin = mean of Sites CL07 and CL08

East Basin = mean of Sites CL09 and CL10

2020 TMDL target for Dissolved Oxygen (DO) is no less than 5 mg/L in the hypolimnion

Bold Underline - Indicates exceedance of TMDL target

Figure 3-12. Mean In-Situ Physical Water Quality Parameters – Canyon Lake Main Basin

(Values represent the mean of Sites CL07 & CL08. Missing epilimnion and hypolimnion values represent time periods when no stratification was present)

Figure 3-13. Mean In-Situ Physical Water Quality Parameters - Canyon Lake East Basin

(Values represent the mean of Sites CL09 & CL10.  Missing epilimnion and hypolimnion values represent time periods when no stratification was present.)

Figure 3-14. Rolling Average Concentrations of Dissolved Oxygen in the Epilimnion and Hypolimnion of Canyon Lake

Means are calculated by averaging the values from all 4 sites of each event with the previous five event values (i.e., one year of data) to obtain a rolling average. Therefore, the graph represents data collected from August 2019 to June 2021. Events in which a thermocline was not present were not included in rolling average.

Figure 3-15. Rolling Average Concentration of Dissolved Oxygen Across the Full Vertical Water Column in Canyon Lake

Each data point is calculated by averaging the values from all 4 sites of each event with the previous five event values (i.e., one year of data) to obtain a rolling average. Therefore, the graph represents data collected from August 2019 to June 2021.

Figure 3-16. In-Situ Water Clarity Using a Secchi Disk– Main and East Basins

#### Analytical Chemistry

Summaries of analytical chemistry concentrations for each monitoring event in Canyon Lake are presented in Tables 3-15 and 3-16. A summary of analytical chemistry mean statistics for each site across the entire monitoring period are presented in Tables 3-17 through 3-19. Concentrations of analytes are presented graphically in Figures 3-17 and 3-18.

Depth-integrated (water column average) concentrations of total nitrogen in the Main Basin (at sites CL07 and CL08) ranged from 1.1 to 2.6 mg/L across the six sampling events, with an annual mean of 1.6 mg/L (up from the 2019-20 annual mean of 1.3 mg/L).  Total nitrogen concentrations at the two East Basin sites ranged from 0.77 to 3.2 mg/L across the six sampling events, with the annual mean of 1.7 mg/L (the same as the 2019-20 annual mean). The total nitrogen concentrations in both basins exhibited a decreasing trend across the monitoring year.  However, the rolling average for total nitrogen ranging from 1.51 to 1.75 mg/L, showed an increasing trend, with all points exceeding the current 2020 TMDL target of 0.75 mg/L (Figure 3-19).

Depth-integrated concentrations of total phosphorus in the Main Basin exhibited a sharp decline from August to December, and then a slight gradual increase across the remainder of the monitoring year. Total phosphorus concentrations ranged from 0.036 to 0.27 mg/L, with an annual mean of 0.096 mg/L (a decrease from the 2019-20 annual mean of 0.12 mg/L).  Total phosphorus concentrations in the East Basin remained steady from August to December, and then exhibited a gradual increase from December to April, before decreasing in June 2021.  Total phosphorus concentrations in the East Basin ranged from 0.046 to 0.24 mg/L, with an annual mean of 0.099 mg/L (a decrease from the 2019-20 annual mean of 0.14 mg/L).  The rolling average for total phosphorus across all sites in the East Basin ranged from 0.10 to 0.14 mg/L, with a decrease towards the end of the monitoring year (Figure 3-19).

Depth-integrated concentrations of total ammonia observed in the Main Basin were at their highest during the first monitoring event of the year (August 2020) at 1.5 mg/L, and then displayed a consistent decrease through February 2021, before increasing in April and June 2021.  This pattern is likely tied to the annual stratification cycle of the lake and was similar to that exhibited by total phosphorus in the Main Basin.  The Main Basin exhibits a stable stratification beginning in early spring (April) through early Fall (October).  During this time low dissolved oxygen in the hypolimnion facilitates the release of phosphorus and ammonia from the sediments.  Total ammonia concentrations in the Main Basin ranged from 0.45 to 2.1 mg/L among the two Main Basin sites, with an annual mean of 0.97 mg/L.  Total ammonia values in the East Basin ranged from non-detect (<0.012) to 2.3 mg/L among the two sites, with an annual mean of 0.69 mg/L. Concentrations in the East Basin were a bit variable, with an initial increase in total ammonia, followed by a decline over the next two events, and then fluctuating concentrations during the last two events of the monitoring year.  Two individual samples, both at Site CL09 (August and October) exceeded the calculated total ammonia CCC value for the protection of aquatic life.  No samples exceeded the total ammonia CMC value.

Total dissolved solids concentrations for both basins displayed an increasing trend across monitoring year, which unlike previous years, appeared to be unimpacted by the storm season rain events. The average TDS concentration in the Main Basin ranged from 390 mg/L to 520 mg/L.  The average concentrations of TDS in the East Basin ranged from 400 mg/L to 620 mg/L. None of the TDS concentrations exceeded the Basin Plan water quality objective of 700 mg/L.

Following a small increase in chlorophyll-a depth-integrated concentrations in the Main Basin between August and October, a steady decline was observed through February 2021, before a small increase in April and a larger increase in June. Depth-integrated concentrations in the Main Basin (mean of Sites CL07 and CL08) across all six sampling events ranged from 1.6 to 36 µg/L, with a mean of 17 µg/L (Figure 3-18).  Depth-integrated concentrations of chlorophyll-a in the East Basin (Sites CL09 and CL10) decreased across the first three events (from 45 to 21 µg/L), and then exhibited a large spike in February 2021 up to 55 µg/L, followed by a large decrease in April.  While the exact reason for this large increase in chlorophyll-a in February is unclear, with the February in-lake monitoring event occurring on February 17, this spike in chlorophyll-a could be due to interference from the large storm event occurring on January 29 - February 1, 2021.  This proposition is backed by the large decrease in chlorophyll-a observed in the subsequent April 2021 monitoring event, dropping to a concentration similar to that observed in the Main Body.  This magnitude of change in chlorophyll-a across two events is unlikely, and the decreasing trend in chlorophyll-may be similar to that observed in the Main Body. The lake-wide chlorophyll-a depth-integrated rolling average remained below the 2020 TMDL target of 25 µg/L for the entire monitoring year (Figure 3-19).

Concentrations of aluminum are measured in Canyon Lake to assess any potential long-term influence from alum additions and relationship to existing water quality objectives. Concentrations of total aluminum ranged from 35 to 64 µg/L in the Main Basin and 63 to 270 µg/L in the East Basin among all sampling locations and dates.  Only one alum application event was performed this monitoring year during the week of October 12-16, 2020.  Concentrations of total aluminum did increase slightly following this application event; however, concentrations were consistently lower overall than the previous monitoring year.  All total aluminum concentrations were well below CCC and CMC values of 1000 and 2300 ug/L total aluminum respectively, based on the US EPA’s Final Aquatic Life Ambient Water Quality Criteria for Aluminum (US EPA, 2018) when using the annual average lake-wide measured pH, and default total organic carbon and hardness values.  Dissolved aluminum concentrations ranged from ND to 34 µg/L in the Main Basin and ND to 93 µg/L in the East Basin.

Table 3-15. Analytical Chemistry Results for Canyon Lake - Monthly Depth-Integrated Results (Aug – Dec 2020)

Notes:

When a concentration was non-detect (ND), the annual mean value for compliance purposes was calculated by converting ND values to zero. If the result of the calculated mean was non-zero, but below the corresponding MDL, the mean value was reported as ND.

a - Total Nitrogen = TKN+NO2+NO3

b - The concentration of unionized ammonia was calculated using equation by Thursby (1986), based on site specific pH and temperature recorded at each location.

c – Sample measured out of holding time

ND – Not detected; NA – Not Applicable/ available; NM:LE – Not measured due to laboratory error

DI = Depth integrated; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; MDL – method detection limit; RL – reporting limit; J - Reported value is an estimate as detection was above the MDL, but below the RL

Bold Underline - Indicates exceedance of TMDL or Basin Plan target

* Exceeds 2004 TMDL Permit NH3 CCC; ** Exceeds 2004 TMDL Permit NH3 CMC

Table 3-16. Analytical Chemistry Results for Canyon Lake- Monthly Depth-Integrated Results (Feb – June 2021)

Notes:

When a concentration was non-detect (ND), the annual mean value for compliance purposes was calculated by converting ND values to zero. If the result of the calculated mean was non-zero, but below the corresponding MDL, the mean value was reported as ND.

a - Total Nitrogen = TKN+NO2+NO3

b - The concentration of unionized ammonia was calculated using equation by Thursby (1986), based on site specific pH and temperature recorded at each location.

c – Sample measured out of holding time

ND – Not detected; NA – Not Applicable/ available

DI = Depth integrated; Surf = Surface 0-2m

µg/L – micrograms per liter; mg/L – milligrams per liter; MDL – method detection limit; RL – reporting limit; J - Reported value is an estimate as detection was above the MDL, but below the RL

Bold Underline - Indicates exceedance of TMDL or Basin Plan target

Table 3-17. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Each Site in the Main Basin

Notes:

When a concentration was non-detect, the annual value for compliance purposes was calculated by converting non-detect (ND) values to zero. If the result of the calculation was below the corresponding MDL, the average value was reported as ND.

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Values calculated using water column mean ammonia, temperature, salinity and pH.  Calculated using equation by Thursby (1986). The range of TMDL target thresholds apply to individual samples, not applicable to annual means.

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

4 - Based on the 2013 EPA Aquatic Life Ambient Water Quality Criteria for Ammonia - Freshwater 2013 (EPA 822-R-18-002). The CMC calculation is assuming the absence of Oncorhynchus spp and is provided for comparison only.

NA – Not applicable/ available; ND – not detected

DI = Depth integrated; Surf = Surface 0-2m

mg/L – micrograms per liter; ug/L – milligrams per liter; MDL – method detection limit; RL – reporting limit; J –Reported value was detected above the MDL, but below the RL

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

Table 3-18. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Each Site in the East Basin

Notes:

When a concentration was non-detect, the annual value for compliance purposes was calculated by converting non-detect (ND) values to zero. If the result of the calculation was below the corresponding MDL, the average value was reported as ND.

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Values calculated using water column mean ammonia, temperature, salinity and pH.  Calculated using equation by Thursby (1986). The range of TMDL target thresholds apply to individual samples, not applicable to annual means.

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

4 - Based on the 2013 EPA Aquatic Life Ambient Water Quality Criteria for Ammonia - Freshwater 2013 (EPA 822-R-18-002). The CMC calculation is assuming the absence of Oncorhynchus spp and is provided for comparison only.

NA – Not applicable/ available; ND – not detected

DI = Depth integrated; Surf = Surface 0-2m

mg/L – micrograms per liter; ug/L – milligrams per liter; MDL – method detection limit; RL – reporting limit; J –Reported value was detected above the MDL, but below the RL

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

Table 3-19. Analytical Chemistry Results for Canyon Lake - Annual Mean Statistics for Both Main and East Basins

Notes:

When a concentration was non-detect, the annual value for compliance purposes was calculated by converting non-detect (ND) values to zero. If the result of the calculation was below the corresponding MDL, the average value was reported as ND.

a - Total Nitrogen = TKN+NO2+NO3

b - Annual average

c - Values calculated using water column mean ammonia, temperature, salinity and pH.  Calculated using equation by Thursby (1986). The range of TMDL target thresholds apply to individual samples, not applicable to annual means.

1 – 2020 TMDL Target, based on Table 5-9n of 2004 TMDL

2 – 2015 TMDL Target, based on Table 5-9n of 2004 TMDL

3 – Santa Ana Region Basin Plan Objective

4 - Based on the 2013 EPA Aquatic Life Ambient Water Quality Criteria for Ammonia - Freshwater 2013 (EPA 822-R-18-002). The CMC calculation is assuming the absence of Oncorhynchus spp and is provided for comparison only.

NA – Not applicable/ available; ND – not detected

DI = Depth integrated; Surf = Surface 0-2m

mg/L – micrograms per liter; ug/L – milligrams per liter; MDL – method detection limit; RL – reporting limit; J –Reported value was detected above the MDL, but below the RL

Bold Underline - Indicates exceedance of Basin Plan/TMDL target

Figure 3-17. Canyon Lake Analytical Chemistry – Depth-Integrated Means

Main Basin values represent the mean of Sites CL07 & CL08, East Basin values represent the mean of Sites CL09 & CL10

Long term trends can be found in Appendix E

Figure 3-18. Canyon Lake Analytical Chemistry – Depth-Integrated Chlorophyll-a

Main Basin values represent the mean of Sites CL07 & CL08, East Basin values represent the mean of Sites CL09 & CL10

Long term trends can be found in Appendix E

Figure 3-19. Canyon Lake Analytical Chemistry- Rolling Averages

Each data point is calculated by averaging the value from each event across all 4 sites with the previous five events across all 4 sites (i.e. one year of data) to obtain a rolling average. Therefore, each graph represents data collected from October 2018 to June 2020.

## Satellite Imagery

Beginning with the 2015-2016 FY, the TMDL Task Force contracted with satellite vendor EOMAP to conduct remote sensing using LandSat and Sentinel-2 satellite imagery to estimate chlorophyll-a and turbidity concentrations in Lake Elsinore and Canyon Lake.  Using 30-m (LandSat) or 10-m (Sentinel-2) pixel resolution, this effort produced maps of the lakes showing graphical, color-coded images of chlorophyll-a and turbidity concentrations at up to approximately 1,000 unique data points across Canyon Lake and approximately 11,000 unique data points across Lake Elsinore.  This tool provides a snapshot of conditions throughout the lakes at a given point in time, as opposed to the single data points provided at water quality collection locations and dates. The satellite images are also able to provide a sense of the relative variability in algae concentrations across the lake that can be rather dramatic and missed by measuring individual values from only a few discrete locations.   However, the satellite imagery only represents approximately the upper 3-feet of the water column depending on water clarity, and therefore cannot completely replace manual sampling where depth-integrated values are required.

As part of the TMDL compliance monitoring, satellite imagery depicting surficial lake-wide chlorophyll-a and turbidity concentrations in Lake Elsinore and Canyon Lake were generated for each in-lake monitoring event.  Satellite images for each lake during the eight monitoring events evaluated in the report are presented in Figures 3-20 through 3-23. Significant spatial variability in chlorophyll-a is evident, providing a more complete assessment of algal density conditions across each lake.

To quantify the data presented in the satellite images, cumulative frequency distribution plots showing lake-wide chlorophyll-a concentrations based on individual pixels from the satellite measurements are provided in Figures 3-24 and 3-25.  Satellite derived mean and median concentrations along with measured in-lake chlorophyll-a concentrations in the surface composite (0-2m) sample are provided for each date showing how these single samples compare to concentrations throughout the entire lake.  Mean and median lake-wide values were derived from satellite imagery data treating each pixel as a unique individual data point.

The satellite images for Lake Elsinore show an initial increase in lake wide chlorophyll-a concentration from July to August 2020, but then a gradual decrease in chlorophyll-a through April 2021, followed by an increase in June 2021.  These generalizations are validated when using satellite pixel data to calculate lake-wide mean and median chlorophyll-a concentrations. The general lake-wide pattern observed using satellite imagery was consistent with measured month-to-month surface chlorophyll-a values at Site LE02.

Chlorophyll-a concentrations in Canyon Lake derived from satellite imagery remained relatively consistently low throughout the monitoring period, with a slight increase in chlorophyll-a in October 2020 in the Main Basin, as well as February and April for the East Basin.  Measured in-lake concentrations of chlorophyll-a in the Main Basin were relatively low and generally matched that seen in the satellite images (< 36 mg/L).  Satellite imagery did show some periods of elevated chlorophyll-a in February and April 2021 for the East Basin, though it should be noted that there may be some edge-interference effects as a result of land and water pixels mixing near the edges of the narrow channel. This “edge effect” is somewhat diminished during non-summer months when Sentinel-2 satellite data is used, which generates a smaller pixel size (10-m) than the LandSat satellite (30-m) used during summer months (June – Sept) reducing the possibility of mixing land and water in a single pixel.  Satellite imagery for February 2020 however was derived from the Sentinel-2 satellite data with greater resolution, reducing interference from edge effects.  However, as mentioned in the analytical chemistry section (Section 3.3.3) for Canyon Lake, there is the possibility that the elevated chlorophyll-a seen in February 2021 for the East Basin could be the result of water quality interference caused by the recent storm event prior to sampling.  Overall, satellite estimates of chlorophyll-a data generally tracked with Canyon Lake in-lake analytical surface sample data.

August 11, 2020

September 20, 2020

February 17, 2021

December 9, 2020

October 10, 2020

July 28, 2020

Figure 3-20.  Satellite Imagery of Chlorophyll-a Concentrations in Lake Elsinore

(Data gaps in August are due to sunglint)

June 2, 2021

April 8, 2021

Figure 3-20 (cont.).  Satellite Imagery of Chlorophyll-a Concentrations in Lake Elsinore

August 11, 2020

July 28, 2020

September 20, 2020

December 9, 2020

February 17, 2021

October 10, 2020

Figure 3-21.  Satellite Imagery of Turbidity Concentrations in Lake Elsinore

(Data gaps in August are due to sunglint)

June 2, 2021

April 8, 2021

Figure 3-21 (cont.).  Satellite Imagery of Turbidity Concentrations in Lake Elsinore

September 20, 2020

August 11, 2020

July 28, 2020

December 9, 2020

February 17, 2021

October 10, 2020

Figure 3-22.  Satellite Imagery of Chlorophyll-a Concentrations in Canyon Lake

(Data gaps in December 2020 are caused by sunglint)

June 2, 2021

April 8, 2021

Figure 3-22 (cont.).  Satellite Imagery of Chlorophyll-a Concentrations in Canyon Lake

September 20, 2020

August 11, 2020

July 28, 2020

February 17, 2021

October 10, 2020

December 9, 2020

Figure 3-23.  Satellite Imagery of Turbidity Measurements Canyon Lake

(Data gaps in December 2020 are caused by sunglint.)

June 2, 2021

April 8, 2021

Figure 3-23 (cont.).  Satellite Imagery of Turbidity Measurements Canyon Lake

Figure 3-24.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Lake Elsinore Relative to Measured Chlorophyll-a in Field Collected Samples

Colored dots represent the in-lake surface (0-2m) analytical measured concentration for each event

Figure 3-25.  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Canyon Lake Relative to Measured Chlorophyll-a in Field Collected Samples

Colored dots represent the in-lake surface (0-2m) analytical measured concentration for each event

Figure 3-25. (cont).  Cumulative Distribution of Satellite Derived Chlorophyll-a Concentrations in Canyon Lake Relative to Measured Chlorophyll-a in Field Collected Samples

Colored dots represent the in-lake surface (0-2m) analytical measured concentration for each event

# References

Amec Foster Wheeler. 2016. Quality Assurance Project Plan for Lake Elsinore, Canyon Lake, and San Jacinto River Watershed TMDL Monitoring Program. May 2016.

Amec Foster Wheeler. 2017. Lake Elsinore and Canyon Lake Watersheds Nutrient TMDL Monitoring, 2016-2017 Annual Report.  Prepared for the Lake Elsinore & San Jacinto Watersheds Project Authority, August 2017.

Haley & Aldrich, Inc. 2016. Lake Elsinore and Canyon Lake Nutrient TMDL Compliance Monitoring Work Plan for Lake Elsinore and San Jacinto Watershed Authority. Riverside, CA. April 2016.

Lake Elsinore and San Jacinto Watersheds Authority (LESJWA). 2006. Lake Elsinore and Canyon Lake Nutrient TMDL Monitoring Plan. CA Regional Water Quality Control Board, Santa Ana Region. February.

Montgomery Watson and Elsinore Valley Municipal Water District (MWH/ EVMWD). 2009.  Canyon Lake Bacteria Characterization Study. Technical Memorandum prepared for the Lake Elsinore and Canyon Lake TMDL Task Force.  December 1, 2009

Riverside County Flood Control and Water Conservation District (RCFCWCD). 2020. Hydrologic Data Collection, Rain Fall Reports.  http://content.rcflood.org/RainfallMap/

Santa Ana Regional Water Quality Control Board. 2007. Resolution Amending the Water Quality Control Plan for the Santa Ana River Basin to Incorporate Nutrient Total Maximum Daily Loads (TMDLs) for Lake Elsinore and Canyon Lake, No. R8-2004-0037.

Santa Ana Regional Water Quality Control Board. 2007. Resolution Approving Plans and Schedules Submitted by the Canyon Lake/Lake Elsinore TMDL Task Force and Individual Discharger Groups Pursuant to the Lake Elsinore and Canyon Lake Nutrient Total Maximum Daily Loads Specified in the Water Quality Control Plan for the Santa Ana River Basin. Resolution R8-2007-0083.

Thursby, C.B. 1986. Memorandum to David J. Hansen, U.S. EPA, Narragansett, Rhode Island.

United States Environmental Protection Agency (US EPA). 2018. 2018 Final Aquatic Life Ambient Water Quality Criteria for Aluminum. EPA-822-R-18-001. U.S. EPA Office of Water. December 2018. EPA-822-R-18-001

This page intentionally left blank.

Based on federally listed Riverside fairy shrimp (Streptocephalus woottoni) in adjacent wetlands.

2004-05, 2005-06 and 2008-09 monitoring years each had several months of missing temperature data and were not included in this retrospective analysis. No temperature profile data was collected across the July 2012 to June 2015 monitoring years.

Temperature data was not available prior to the 2007-08 monitoring year. No temperature profile data was collected across the July 2012 to June 2015 monitoring years.

The Sentinel-2 satellite data cannot be used during summer months due to a glare from the sun caused by the angle of satellite viewing, and thereby reducing the image quality.